SCHOOL OF DIGITAL MEDIA AND INFOCOMM TECHNOLOGY (DMIT)

| Date of Submission: | Date of submission |
| --- | --- |

| Prepared for: | Ms Dora Chua |
| --- | --- |

| Class: | DIT/FT/3A/41 |
| --- | --- |

| Submitted by: |  |
| --- | --- |

| Student ID | Name |
| --- | --- |
| 1530317 | Angelica Gonzales |
| 1530320 | Niyati Srinivasan |
| 1543007 | Eunice Poh Peck Rong |

IOT CA2

Step-by-step Tutorial

DIPLOMA IN BUSINESS INFORMATION TECHNOLOGY

DIPLOMA IN INFORMATION TECHNOLOGY

DIPLOMA IN INFOCOMM SECURITY MANAGEMENT

ST0324 Internet of Things (IOT)

2017/2018 Semester 1

- Table of Contents

Section 1 Overview of project	3

A.	Where we have uploaded our tutorial	3

B.	Why have we chosen to upload to this site	3

C.	What have we uploaded	3

D.	What is the application about?	4

AnE Smart Doorbell Systems	4

E.	Summary of the steps that will be described	5

F.	How does the final RPI set-up looks like?	6

G.	How does the web application look like?	7

Welcome	7

Login	7

Register	8

Dashboard	8

Picture	9

Lights	10

Message	11

Section 2 Hardware Requirements	12

Hardware checklist	12

Button	12

LED	12

330Ω Resistor	13

10kΩ Resistor	13

I2C LCD Screen	13

Buzzer	14

Jumper Wires	15

PiCam	15

Section 3 Setup Overview	16

Fritzing Diagram	16

Codes	17

Firebase Console	19

Bluemix Console	19

Section 3 AnE Smart Doorbell Systems (Part 1)	20

Connect the I2C LCD Screen	20

Connect the LED and Resistor	21

Connect the Piezo Buzzer	22

Connect the Button	23

Enable Camera	24

Section 4 Setting up Smart Doorbell (Part 2)	25

Amazon Web Services (AWS) Setup	25

Section 5 AnE Smart Doorbell Systems (Part 3)	43

Installing the necessary libraries	43

ane_doorbell.py	44

Section 6 AnE Smart Doorbell Systems (Part 4)	51

Node.js on Bluemix	51

Code	57

Telegram Bot	86

Firebase Console	89

Publish to Bluemix	94

Section 7 AnE Smart Doorbell Systems (Part 5)	95

Web Application Setup	95

Add AWS Certificates	96

server.py	97

welcome.html	102

login.html	105

register.html	108

dashboard.html	111

lcd.html	116

light.html	122

picture.html	129

welcome.css	136

login.css	141

register.css	146

style.css	153

Section 9	155

Running	155

Web Application	155

Smart Doorbell	155

# Section 1
Overview of project

  - Where we have uploaded our tutorial

Link: https://www.instructables.com/member/NiyatiS5/instructables/

  - Why have we chosen to upload to this site

Instructable is an online community that allows you explore, share and make projects. It has become a known documentation system since 2005 and offers a variety of instructable classes and subjects, ranging from 3D printing to Design. It allows you to add texts, images, and files, which ensures your documents are of an expected quality before you publish your built projects, demonstrate tutorials, solicit feedback or market projects.

  - What have we uploaded

Instructions and images for better understanding as been uploaded to instructables.

  - What is the application about?

### AnE Smart Doorbell Systems

AnE Smart Doorbell Systems is a system that aims to inform you when someone presses the button of your doorbell. When a guest presses the button of the doorbell, which is placed outside the house, the buzzer will ring. The PiCam, which is mounted outside the house, allows the them to take a photo/ short video should they feel that there are potential intruder(s).

The LED is placed inside the house to allow the homeowner(s) to simulate that the house is not vacant and ward away potential intruders.

To differentiate it from your standard doorbell system, it makes use of Telegram messenger application to notify the home owner(s) when a guest presses the button and allow the home owner(s) to send a response. There are various types of responses that the homeowner(s) can send- display a message via LCD screen, turn on LED or take a video/ picture using the PiCam. Should they not have a Telegram account linked to their AnE Smart Doorbell Systems’ account, they can make use of the web interface which allows them to perform the same functions as if they were to use Telegram.

  - Summary of the steps that will be described

Provide a bullet list of the steps that will be covered in the other parts of this tutorial

|  | Section | Description |
| --- | --- | --- |
|  | Overview | Provides an overview of what you will be doing to make this application. |
|  | Hardware Requirements | Provides an overview of hardware components required. |
|  | Setup Overview | Shows the expected outcome after setting up the application |
| Sections 4 to 8 provides the step-by-step instructions to set up the application |  |  |
|  | AnE Smart Doorbell Systems (Part 1) | Provides a step-by-step tutorial on setting up your hardware components. |
|  | AnE Smart Doorbell Systems (Part 2) | Provides a step-by-step tutorial on setting up your AWS IoT Console |
|  | AnE Smart Doorbell Systems (Part 3) | Provides you with the commands to install necessary libraries to set up your AnE Smart Doorbell System functions. |
|  | AnE Smart Doorbell Systems (Part 4) | Provides you with a step-by-step tutorial to setting up your Node.js Server. |
|  | AnE Smart Doorbell Systems (Part 5) | Provides a step-by-step tutorial on setting up the website |
|  | Running the System | Steps to run the Website and the Product |

  - How does the final RPI set-up looks like?

  - How does the web application look like?

### Welcome

### Login

### Register

### Dashboard

### Picture

### Lights

### Message

# Section 2
Hardware Requirements

- Hardware checklist

Overview of the hardware components required for this application

### Button

| Tactile Push-button |  |
| --- | --- |
| The button contains 4 legs where signals are sent. The push buttons act as a sensor and we can determine whether it is pressed or not pressed through its High or Low state passed through the legs In this Smart Doorbel application, the Button will represents the Doorbell where visits press on it. | Button Quantity: 1 |

### LED

| Light-emitting diode |  |
| --- | --- |
| LEDs consists of two legs where the longer leg (anode) goes to the positive voltage and the shorter leg (cathode) goes to the negative side of the circuit The LEDs in Smart Doorbell are placed behind the doors for users to simulate that someone is at home. | LED Quantity: 1 |

### 330Ω Resistor

| 330ohm resistor |  |
| --- | --- |
| Resistors must always be used when connecting LEDs to limit the amount of current flowing through We will be using a 300Ω resistor which can be identified by its colour band: Orange, Orange, Brown and Gold | 330Ω Resistors Quantity: 1 |

### 10kΩ Resistor

| 10k ohm Resistor |  |
| --- | --- |
| To moderate the flow of current for the LDR circuit, we will be using a 10kΩ resistor which can be identified by its colour band: Brown, Black, Orange and Gold | 10kΩ Resistors Quantity: 1 |

### I2C LCD Screen

| LCD Screen |  |
| --- | --- |
| we will use the commonly available 16x2 LCD which can display up to 32 characters. We will use the i2C version which require you to connect only 2 GPIO pins to your Raspberry Pi. In this Smart Doorbell application, the LCD screen will show the users message to the guests at the door. | LCD Screen Quantity: 1 |

### Buzzer

| Buzzer |  |
| --- | --- |
| A buzzer is an audio signaling device which is commonly found in circuits to create a buzzing or beeping noise. Buzzers can be categorized as active buzzers and passive ones. An active buzzer can be connected just like an LED but they are even easier to use because a resistor is not needed. A buzzer typically has 2 pins o VOUT – Connect this to a GPIO pin to control its value o GND – Connect this to ground | Buzzer Quantity: 1 |

### Jumper Wires

| Jumper Wires |  |
| --- | --- |
| Jumper wires are electrical wires which are needed to connect the components together. | M/F Jumper Wires Quantity: 4 M/M Jumper Wires Quantity: 8 |

### PiCam

| PiCam |  |
| --- | --- |
| PiCam allows the homeowner(s) to take a photo or a video to monitor for potential intruders | PiCam Quantity: 1 |

# Section 3
Setup Overview

- Fritzing Diagram

This is how your final set-up would look like.

- Codes

At the end of the setup, you should have the following folders created for each component, and the directory tree should look like this

Directory for Raspberry Pi Codes

. ¦ ├── certs ¦ ├── certificate.pem.crt ¦ ├── private.pem.key ¦ ├── public.pem.key ¦ └── rootca.pem ├── pictures ├── videos ├── ane_doorbell.py └── awscredentials.py

Directory for Node.js Server Codes

. ¦ ├── .git ¦ ├── APIs ¦ ├── email.js ¦ ├── product.js ¦ ├── pubsub.js ¦ └── user.js ¦ ├── certs ¦ ├── certificate.pem.crt ¦ ├── private.pem.key ¦ ├── public.pem.key ¦ └── rootca.pem ¦ ├── Controllers ¦ ├── DeviceAPIController.js ¦ ├── ProductAPIController.js ¦ └── UserAPIController.js ¦ ├── node_modules ¦ ├── .cfignore ├── .gitignore ├── .project ├── app.js ├── config.js ├── LICENSE ├── manifest.yml ├── package.json └── README.md

Directory for Website Code

. ¦ ├── certs ¦ ├── certificate.pem.crt ¦ ├── private.pem.key ¦ ├── public.pem.key ¦ └── rootca.pem ¦ ├── static ¦ └── css ¦ ├── login.css ¦ ├── register.css ¦ ├── styles.css ¦ └── welcome.css ¦ ├── templates ¦ ├── dashboard.html ¦ ├── lcd.html ¦ ├── light.html ¦ ├── login.html ¦ ├── picture.html ¦ ├── register.html ¦ └── welcome.html ¦ └── server.py

- Firebase Console

- Bluemix Console

# Section 3
AnE Smart Doorbell Systems (Part 1)

- Connect the I2C LCD Screen

Connect LCD Screen

- Connect the pins on the LCD to the RPI as follows

- Install the rpi-lcd library using the commands below sudo pip install rpi-lcd

- Final Setup

- Connect the LED and Resistor

Connect LED and Resistor

- Plug in the longer end of the LED closer to the T-Cobbler Kit

- Connect the longer end of the led using an blue jump wire to the RPi pin as follows:

- Connect the one end of the 330 Ω resistor to the negative bus strip. - Connect the other end of the 330 Ω resistor in the same column as the shorter end of the LED - Connect one end of the black jumper wire to the GND and the other to the negative bus strip that the resistor is connected

- Connect the Piezo Buzzer

Steps

- Connect the Buzzer with the RPi as follows:

- Connect the Button

Steps

- Place the button on the breadboard as shown:

- Enable Camera

Enable Camera- using Graphical User Interface (GUI)

1. Open the Raspberry Pi Configuration Tool from the main menu

2. Select Interfaces, enable Camera and click OK

# Section 4
Setting up Smart Doorbell (Part 2)

- Amazon Web Services (AWS) Setup

AnE Smart Doorbell Systems makes use of Amazon Web Services (AWS) to publish and subsribe to the state of the hardware components, such as LCD Screen and PiCam.

When the button is pressed, it publishes a payload, which informs subscribers such Telegram or web app. The other hardware components also publishes payloads, which keeps the user informed of its state and keeps track of its history.

Sign In to the AWS IoT Console

- At the AWS Console, click on Services

- Select AWS IoT You will be navigated here

Create & Register Thing

- At the AWS IoT Dashboard, select Registry ¦ Things

- Click on Create *Note: If you don’t have a thing yet, click on Register a thing.

- Give you Thing a name and click Create thing

- Select Interact and copy and paste REST API Endpoint to a notepad

Create Certificates

- Click the back arrow to go back to AWS IoT Dashboard

- At the AWS IoT Dashboard, select Security ¦ Certificates

- Click on Create *Note: If you don’t have a certificate yet, click on Create certificate

- Click Download for each certificate

- When you click Download for your root CA, right click and save as rootca.pem

- Move your certs to a new folder named certs and rename them accordingly

- Click Activate

Create Security Policy

1. Click Attach a policy

2. Click Create a new policy

3. Define the policy name and authorized actions

4. Click Create

Attach Security Policy and Thing to Certificates

1. Click the back arrow to go back to AWS IoT Dashboard

2. At the AWS IoT Dashboard, select Security ¦ Certificates

3. Click on the Certificate menu and select Attach policy

4. Select the policy you created and click Attach

5. Click on the Certificate menu and select Attach thing

6. Select the thing you created and click Attach

Create A Folder

- Open a terminal in your raspberryPi and run mkdir ~/smartdoorbell

2. Transfer certs folder to your raspberryPi and put it inside ~/smartdoorbell folder

smartdoorbell

. ¦ └── certs ├── certificate.pem.crt ├── private.pem.key ├── public.pem.key └── rootca.pem

awscredentials.py

- Create smartdoorbell/awscredentials.py to store your AWS Credentials

- Copy and Paste the following codes accessKey = '' #specify your AWS access key secretAccessKey = '’ #specify your AWS secret access key.

- Your directory should look like this now

. ¦ ├── certs ¦ ├── certificate.pem.crt ¦ ├── private.pem.key ¦ ├── public.pem.key ¦ └── rootca.pem └── awscredentials.py

# Section 5
AnE Smart Doorbell Systems (Part 3)

- Installing the necessary libraries

Steps

1. Install Flask with this command sudo pip install flask

2. Install the AWS Python library with this command sudo pip install AWSIoTPythonSDK

3. Type the following command to install Boto, the Python library for AWS on your raspberrypi sudo pip install boto3

4. Type the following command to install the AWS Command‐Line Interface Client on your Raspberry Pi sudo pip install awscli

5. Create a python script ane_doorbell.py with the code below sudo nano ~/smartdoorbell/ane_doorbell.py.

Create folders

- Open a terminal in your raspberryPi and run mkdir ~/smartdoorbell/pictures

- Open a terminal in your raspberryPi and run mkdir ~/smartdoorbell/videos

- ane_doorbell.py

Steps

- Create a python script ane_doorbell.py with the code below sudo nano ~/smartdoorbell/ane_doorbell.py.

- Copy the following codes # coding: utf8 from AWSIoTPythonSDK.MQTTLib import AWSIoTMQTTClient from gpiozero import Button, Buzzer, LED from rpi_lcd import LCD from picamera import PiCamera from signal import pause from datetime import datetime import calendar from time import sleep import json import boto3 import botocore import time import subprocess import shlex import os import awscredentials camera = PiCamera() bz = Buzzer(6) led = LED(18) lcd = LCD() button = Button(5, pull_up=False) # Set the filename and bucket name bucket_name = 'anesmartdoorbell' # replace with your own unique bucket name region = 'ap-southeast-1' exists = True spam = False start = time.time() host = "a3t0nm0d0jd547.iot.us-west-2.amazonaws.com" rootCAPath = "certs/rootca.pem" certificatePath = "certs/certificate.pem.crt" privateKeyPath = "certs/private.pem.key" accessKey = awscredentials.accessKey secretAccessKey = awscredentials.secretAccessKey productKey = "A1B2C3" # Create an S3 resource s3 = boto3.resource('s3', aws_access_key_id=accessKey, aws_secret_access_key=secretAccessKey) def onLight(): led.on() def offLight(): led.off() def lcdWrite(message): lcd.text(message[:16], 1) lcd.text(message[16:32], 2) def lcdClear(): lcd.clear() def doorbellPressed(): global productKey, start, spam bz.on() if (spam == False): takePicture() start = time.time() d = datetime.utcnow() timestamp = calendar.timegm(d.utctimetuple()) payload = json.dumps({ "device": "button", "productKey": productKey, "action": "button_pressed", "data": { "datetime": timestamp } }) my_rpi.publish("anedingding/" + productKey + "/button", payload, 1) spam = True else: difference = time.time() - start if(difference > 10): spam = False start = time.time() def doorbellRelease(): bz.off() def takePicture(): global region, productKey, bucket_name print("taking picture") d = datetime.utcnow() timestamp = calendar.timegm(d.utctimetuple()) file_name = "{}.jpg".format(timestamp) camera.capture("pictures/" + file_name) full_path = "{}/{}".format(productKey, file_name) # Upload picture to s3 s3.Bucket(bucket_name).put_object(Key=full_path, Body=open("pictures/" + file_name, 'rb')) picturePath = "".join( ["https://s3-", region, ".amazonaws.com/", bucket_name, "/", full_path]) payload = json.dumps({ "device": "camera", "productKey": productKey, "action": "picture_taken", "data": { "datetime": timestamp, "picturePath": picturePath } }) my_rpi.publish("anedingding/" + productKey + "/camera", payload, 1) print("File uploaded") def takeVideo(): global region, productKey, bucket_name d = datetime.utcnow() timestamp = calendar.timegm(d.utctimetuple()) file_name = "{}.h264".format(timestamp) new_file_name = "{}.mp4".format(timestamp) full_path = "{}/{}".format(productKey, new_file_name) camera.start_recording("videos/" + file_name) sleep(8) camera.stop_recording() video_save_path = os.path.dirname(os.path.realpath( __file__)) + "/videos/{}".format(file_name) # Convert video to mp4 format from subprocess import CalledProcessError command = "MP4Box -add {} videos/{}".format(video_save_path, new_file_name) try: output = subprocess.check_output( command, stderr=subprocess.STDOUT, shell=True) subprocess.Popen("rm {}".format(video_save_path), shell=True) # Delete the h264 video except CalledProcessError as e: print('FAIL:\ncmd:{}\noutput:{}'.format(e.cmd, e.output)) # Upload video to s3 s3.Bucket(bucket_name).put_object(Key=full_path, Body=open("videos/" + new_file_name, 'rb')) videoPath = "".join( ["https://s3-", region, ".amazonaws.com/", bucket_name, "/", full_path]) payload = json.dumps({ "device": "camera", "productKey": productKey, "action": "video_taken", "data": { "datetime": timestamp, "videoPath": videoPath } }) my_rpi.publish("anedingding/" + productKey + "/camera", payload, 1) def customCallback(client, userdata, message): global productKey print("Received a new message: ") print(json.loads(message.payload)) print("from topic: {}".format(message.topic)) print("‐‐‐‐‐‐‐‐‐‐‐‐‐‐\n\n") payloadData = json.loads(message.payload) action = payloadData["action"] device = payloadData["device"] if (device == "led"): if(action == "on_led"): onLight() payload = json.dumps({ "device": "led", "productKey": productKey, "action": "led_on" }) my_rpi.publish("anedingding/" + productKey + "/led", payload, 1) elif (action == "off_led"): offLight() payload = json.dumps({ "device": "led", "productKey": productKey, "action": "led_off" }) my_rpi.publish("anedingding/" + productKey + "/led", payload, 1) elif (device == "lcd"): if (action == "display_message"): message = str(payloadData["data"]["message"]) lcdWrite(message) payload = json.dumps({ "device": "lcd", "productKey": productKey, "action": "message_display", "data": { "message": message, "datetime": str(payloadData["data"]["datetime"]) } }) my_rpi.publish("anedingding/" + productKey + "/lcd", payload, 1) elif (action == "clear_message"): lcdClear() payload = json.dumps({ "device": "lcd", "productKey": productKey, "action": "message_clear" }) my_rpi.publish("anedingding/" + productKey + "/lcd", payload, 1) elif (device == "camera"): if (action == "take_picture"): takePicture() elif (action == "take_video"): takeVideo() try: s3.meta.client.head_bucket(Bucket=bucket_name) except botocore.exceptions.ClientError as e: error_code = int(e.response['Error']['Code']) if error_code == 404: exists = False if exists == False: # Create the bucket policy bucket_policy = json.dumps({ "Version": "2008-10-17", "Statement": [ { "Sid": "AllowPublicRead", "Effect": "Allow", "Principal": { "AWS": "*" }, "Action": "s3:GetObject", "Resource": "arn:aws:s3:::%s/*/*" % bucket_name } ] }) s3.create_bucket(Bucket=bucket_name, CreateBucketConfiguration={ 'LocationConstraint': region}) # Create an S3 client s3client = boto3.client('s3', aws_access_key_id=accessKey, aws_secret_access_key=secretAccessKey) # Set the new policy on the given bucket s3.put_bucket_policy(Bucket=bucket_name, Policy=bucket_policy) print("S3 bucket created") my_rpi = AWSIoTMQTTClient(productKey) my_rpi.configureEndpoint(host, 8883) my_rpi.configureCredentials(rootCAPath, privateKeyPath, certificatePath) my_rpi.configureOfflinePublishQueueing(-1) # Infinite offline Publish queueing my_rpi.configureDrainingFrequency(2) # Draining: 2 Hz my_rpi.configureConnectDisconnectTimeout(10) # 10 sec my_rpi.configureMQTTOperationTimeout(5) # 5 sec # Connect and subscribe to AWS IoT my_rpi.connect() my_rpi.subscribe("anedongdong/" + productKey + "/#", 1, customCallback) print("Subscribed") button.when_pressed = doorbellPressed button.when_released = doorbellRelease pause().

Task

- Create 2 folders pictures and videos in your project directory for storing the pictures and videos taken using the PI Camera

- You project directory should look like this now . ¦ ├── certs ¦ ├── certificate.pem.crt ¦ ├── private.pem.key ¦ ├── public.pem.key ¦ └── rootca.pem ├── pictures ├── videos ├── ane_doorbell.py └── awscredentials.py

# Section 6
AnE Smart Doorbell Systems (Part 4)

- Node.js on Bluemix

Overview

- Create a Bluemix Cloud Foundry App - Enable Continuous Integration - Clone the files in Git to your computer

Task

- In you Bluemix Dashboard, click on Create App

- Search “SDK for Node.js” and click on SDK for Node.js under the Cloud Foundry Apps

- Key in the App name for your App and click Create

- After the App has been created, scroll down to Continuous delivery under Overview tab and click Enable

- Ensure that the settings are the same and click Create

- At the Overview of your Toolchains, click on Git

- You will be navigated to your project in Git. Click on the Blue pattern circle on the top right hand corner and click on Settings

- In the Access Token tab create a Personal Access Token for yourself to clone the project

- Copy the generated Access Token in a notepad, we wil be using it when cloning the project later

- Copy your Git URL and take note of the username below

- Open your command prompt in the directory you want to save the project in and type in git clone <project url> Press enter and you will be prompted to key in the Git Credentials. Enter the UserName from the Git Console and the generated Personal Access Token as the Password

- Done the Node.js project is now in your project directory

### Code

The Project you clone should look like this

. ¦ ├── .git ├── node_modules ├── public ¦ ├── images ¦ ¦ └── newapp-icon.png ¦ ├── stylesheets ¦ ¦ └── style.css ¦ └── index.html ├── .cfignore ├── .gitignore ├── .project ├── app.js ├── config.js ├── LICENSE ├── manifest.yml ├── package.json └── README.md

Task

- Open the project in a text editor such as Visual Studio Code or Sublime

- Replace the package.json file with the codes below { "name": "test", "version": "0.0.0", "private": true, "scripts": { "start": "node app" }, "dependencies": { "aws-iot-device-sdk": "^2.0.1", "body-parser": "~1.16.0", "cors": "^2.8.4", "express": "~4.14.1", "firebase": "^4.2.0", "moment": "^2.18.1", "node-telegram-bot-api": "^0.28.0", "nodemailer": "^4.0.1", "ws": "^3.1.0" } }.

- Run npm install –save to install the packages needed for this project

- Replace the app.js file with the codes below var express = require('express'); var path = require('path'); var bodyParser = require('body-parser'); var cors = require('cors'); var firebase = require('firebase'); var app = express(); // For BodyParser app.use(bodyParser.json()); app.use(bodyParser.urlencoded({ extended: false })); app.use(cors()); var port = process.env.VCAP_APP_PORT ¦¦ 3000; // Initialise Firebase var firebaseCredentials = require(path.resolve('./config.js')).firebaseCredentials; firebase.initializeApp(firebaseCredentials); // Routes var user = require('./Controllers/UserAPIController'); var product = require('./Controllers/ProductAPIController'); var device = require('./Controllers/DeviceAPIController'); app.use('/api/user', user); app.use('/api/product', product); app.use('/api/device', device); // AWS MQTT and Telegram listener require(path.resolve('./APIs/pubsub')); // Catch 404 and forward to error handler app.use(function (req, res, next) { var err = new Error('Not Found'); err.status = 404; next(err); }); app.listen(port); console.log('Magic happens on port ' + port); module.exports = app;.

- Create a folder call certs for storing the AWS certificates from the AWS IOT Console. Your directory for the certs should look like this . ¦ └── certs ├── certificate.pem.crt ├── private.pem.key ├── public.pem.key └── rootca.pem.

Task

- Create a Controllers folder where all the API routes will be. . ¦ └── Controllers.

- Create a javascript file DeviceAPIController.js and paste the code inside the file var express = require('express'); var router = express.Router(); var path = require('path'); var user = require(path.resolve('./APIs/user.js')); /** * [GET] * Return list of photos taken * http://localhost:8680/api/device/photo */ router.get('/photo', function (req, res) { user.getUserPhoto(req.headers.userkey).then(function (result) { res.send({ success: true, data: result }) }) }) /** * [GET] * Return list of videos taken * http://localhost:8680/api/device/video */ router.get('/video', function (req, res) { user.getUserVideo(req.headers.userkey).then(function (result) { res.send({ success: true, data: result }) }) }) /** * [GET] * Return list of button pressed info * http://localhost:8680/api/device/button */ router.get('/button', function (req, res) { user.getUserButton(req.headers.userkey).then(function (result) { res.send({ success: true, data: result }) }) }) /** * [GET] * Return list of displayed messages * http://localhost:8680/api/device/message */ router.get('/message', function (req, res) { user.getUserMessages(req.headers.userkey).then(function (result) { res.send({ success: true, data: result }) }) }) /** * [GET] * Return status of led * http://localhost:8680/api/device/led */ router.get('/led', function (req, res) { data.getUserLight(req.headers.userkey).then(function (result) { res.send({ success: true, data: result }) }) }) module.exports = router.

- Create a javascript file ProductAPIController.js and paste the code inside the file var express = require('express'); var router = express.Router(); var path = require('path'); var product = require(path.resolve('./APIs/product.js')); /** * [POST] * Add Product Key to firebase * http://localhost:3000/api/product/ * Body: JSON(application/json) * { "productKey": "" } */ router.post('/', function (req, res) { var productKey = req.body.productKey; product.addProductKey(productKey) .then(function (result) { res.send({ success: true, message: result }); }).catch(function (error) { res.send({ success: false, message: error }); }); }); module.exports = router;.

- Create a javascript file UserAPIController.js and paste the code inside the file var express = require('express'); var router = express.Router(); var path = require('path'); var user = require(path.resolve('./APIs/user.js')); var config = require('../config').others; router.get('/', function (req, res) { res.send('Available') }) /** * [POST] * Create Account for Users using Firebase Authentication * http://localhost:3000/api/user/signup * Body: JSON(application/json) * { "email": "", "password": "", "productKey": "" } */ router.post('/signup', function (req, res) { var email = req.body.email; var password = req.body.password; var productKey = req.body.productKey; console.log(productKey) user.signup(email, password, productKey) .then(function (result) { res.send({ success: true, message: result }); }).catch(function (error) { res.send({ success: false, message: error }); }); }); /** * [POST] * Sign in using Firebase Authentication * http://localhost:3000/api/user/signin * Body: JSON(application/json) * { "email": "", "password": "" } */ router.post('/signin', function (req, res) { var email = req.body.email; var password = req.body.password; user.signin(email, password) .then(function (result) { res.send({ success: true, message: result.message, userKey: result.userid, productKey: result.productkey }); }).catch(function (error) { res.send({ success: false, message: error }); }); }); /** * [GET] * URL that user receive in their email to verify their telegram account * http://localhost:8680/verifyemail?productId=&chatId= */ router.get('/verifyuser?', function (req, res) { var productId = req.query.productId; var chatId = req.query.chatId; user.updateTelegramId(productId, chatId) .then(function (result) { res.send(result); }) }); module.exports = router;.

Task

- Create a APIs folder where all the methods will be. . ¦ └── APIs.

- Create a javascript file email.js and paste the code inside the file This file contains the code to send email to user via gmail var email = module.exports = {}; var path = require("path"); var nodemailer = require("nodemailer"); var gmailConfig = require(path.resolve('./config')).gmail; // create reusable transporter object using the default SMTP transport let transporter = nodemailer.createTransport({ service: 'Gmail', auth: { user: gmailConfig.email, pass: gmailConfig.password } }); // create reusable transporter object using the default SMTP transport email.sendEmail = function (email, productId, chatId) { return new Promise(function (resolve, reject) { var host = "https://anesmartdoorbell.au-syd.mybluemix.net" link = host + "/api/user/verifyuser?productId=" + productId + "&chatId=" + chatId; let mailOptions = { to: email, subject: 'Link Telegram Account', // Subject line html: 'Hi ' + email + ',<br><br>' + '<p>Please Click on the link below to link your Product to your Telegram account.<br>' + '<a href=' + link + '>Link telegram account</a></p><br><br>' + 'Best Regards,<br><br>' + 'AnE Smart Doorbell Team' }; // send mail with defined transport object transporter.sendMail(mailOptions, (error, info) => { if (error) { console.log(error); reject(error.toString()) } resolve("Successfully sent email"); }); }); }.

- Create a javascript file product.js and paste the code inside the file They file contains the methods related to Fireabse products node var product = module.exports = {}; var path = require('path'); var firebase = require('firebase'); var rootRef = firebase.database().ref('aneSmartDoorbell'); var productRef = rootRef.child("products"); // add new product key to firebase db product.addProductKey = function (key) { return new Promise(function (resolve, reject) { var newProduct = { "productKey": key, "userKey": "" }; productRef.push(newProduct, function (error) { if (error) { console.log("Error: " + error); reject(error); } else { resolve("Product Key added"); } }); }); } // link the product key to a user product.linkProductToUser = function (productKey, userKey) { productRef.once('value', function (snapshot) { snapshot.forEach(function (oneProduct) { if (oneProduct.val().productKey == productKey) { productRef.child(oneProduct.key).child("userKey").set(userKey); } }); }); } // Check whether product key is already linked to a user product.checkProductKey = function (productKey) { return new Promise(function (resolve, reject) { productRef.once('value', function (snapshot) { var available = "Product Key invalid"; snapshot.forEach(function (oneProduct) { if (oneProduct.val().productKey == productKey) { available = "That Product is already registered under another account"; if (oneProduct.val().userKey == "") { available = true; } } }); resolve(available); }); }); }.

- Create a javascript file pubsub.js and paste the code inside the file This file contains the code that publish and subscribe to AWS IoT and Telegram Bot var pubsub = module.exports = {}; var awsIot = require('aws-iot-device-sdk'); var TelegramBot = require('node-telegram-bot-api'); var path = require("path"); var firebase = require('firebase'); var config = require(path.resolve('./config')).aws; var telegramToken = require(path.resolve('./config.js')).telegram.token; var email = require(path.resolve('./APIs/email.js')); var user = require(path.resolve("./APIs/user.js")); var product = require(path.resolve("./APIs/product.js")); var telegram = new TelegramBot(telegramToken, { polling: true }); var rootRef = firebase.database().ref('aneSmartDoorbell'); var userRef = rootRef.child('users'); var productRef = rootRef.child('products'); // send telegram message pubsub.sendMessage = function (chatId, message, options) { telegram.sendMessage(chatId, message, options) .catch(function (error) { console.log(error) }); } //Listen for any kind of message.There are different kinds of //messages. telegram.on('message', (msg) => { var chatId = msg.chat.id; var userMsg = msg.text; if (userMsg.match(/\/link (.+)¦\/link/)) { // check product key availble and link it to user by sending an email to them var productKey = userMsg.match(/\/link (.+)¦\/link/)[1]; if (productKey) { product.checkProductKey(productKey) .then(function (available) { if (available == true) { // no user link to product pubsub.sendMessage(chatId, 'An account have not been created for that ProductKey. Please create an account before linking your telegram account to the product.', {}); } else if (available == "Product Key invalid") { //no such product keys pubsub.sendMessage(chatId, available, {}); } else { // product key is linked to a user userRef.on('value', function (snapshot) { snapshot.forEach(function (oneUser) { if (oneUser.val().productKey == productKey) { if (oneUser.val().telegramChatId == "") { //send email email.sendEmail(oneUser.val().email, productKey, chatId).then(function () { pubsub.sendMessage(chatId, 'A verification email have been sent to the email linked to that ProductKey', {}); }) } else { pubsub.sendMessage(chatId, 'That Product is already linked to another Telegram Account', {}); } } }) }) } }) } else { pubsub.sendMessage(chatId, 'Try /link <Product Key>', {}); } } else { user.checkTelegramId(chatId).then(function (isLink) { // check if user is already a user if (isLink) { if (userMsg.match(/\/start/)) { pubsub.sendMessage(chatId, 'Hi, I am AnE Smart Doorbell Bot.\n\n' + 'Link your Smart Doorbell Product to your telegram account:' + '\n/link <ProductKey>\n\n' + 'On Lights:\n/onlight\n\n' + 'Off Lights:\n/offlight\n\n' + 'Display Message: (32 char limit)\n/displaymessage <message>' + '\n\nClear Message:\n/clearmessage' + '\n\nTake Photo:\n/picture' + '\n\nTake a 8s Video:\n/video', {}); } else if (userMsg.match(/\/picture¦Take Picture/i)) { // publish to take a picture user.getProductKey(chatId).then(function (productKey) { var data = { "device": "camera", "productKey": productKey, "action": "take_picture" } device.publish('anedongdong/' + productKey + "/camera", JSON.stringify(data)); pubsub.sendMessage(chatId, "Taking photo...", {}); }) } else if (userMsg.match(/\/onlight¦On Light/i)) { // publish to on led user.getProductKey(chatId).then(function (productKey) { var data = { "device": "led", "productKey": productKey, "action": "on_led" } device.publish('anedongdong/' + productKey + "/led", JSON.stringify(data)); pubsub.sendMessage(chatId, "Turning On Light..", {}); }) } else if (userMsg.match(/\/offlight¦Off Light/i)) { // publish to off led user.getProductKey(chatId).then(function (productKey) { var data = { "device": "led", "productKey": productKey, "action": "off_led" } device.publish('anedongdong/' + productKey + "/led", JSON.stringify(data)); pubsub.sendMessage(chatId, "Turning Off Light..", {}); }) } else if (userMsg.match(/\/displaymessage (.+)¦Display a Message¦\/displaymessage¦display message/i)) { // publish to display message var message = userMsg.match(/\/displaymessage (.+)¦Display a Message¦\/displaymessage¦display message/i)[1]; if (message) { user.getProductKey(chatId).then(function (productKey) { var datetime = Math.round((new Date()).getTime() / 1000) var data = { "device": "lcd", "productKey": productKey, "action": "display_message", "data": { "message": message, "datetime": datetime } } device.publish('anedongdong/' + productKey + "/lcd", JSON.stringify(data)); pubsub.sendMessage(chatId, "Displaying Message", {}); }) } else { pubsub.sendMessage(msg.chat.id, "Ok! Please key in the display message in this format: (32 Char limit) /displaymessage <Your Message>", {}); } } else if (userMsg.match(/\/dosomething/i)) { // display list of things the user can do let replyOptions = { reply_markup: { resize_keyboard: true, one_time_keyboard: true, keyboard: [ ['Take Picture'], ['Take Video'], ['Display a Message'], ['Clear Message'], ['On Light'], ["Off Light"] ], }, }; pubsub.sendMessage(chatId, "What do you want to do?", replyOptions); } else if (userMsg.match(/\/video¦Take Video/i)) { // publish to take a video user.getProductKey(chatId).then(function (productKey) { var data = { "device": "camera", "productKey": productKey, "action": "take_video" } device.publish('anedongdong/' + productKey + "/camera", JSON.stringify(data)); pubsub.sendMessage(chatId, "Taking Video...", {}); }) } else if (userMsg.match(/\/clearmessage¦Clear Message/i)) { // publish to clear lcd message user.getProductKey(chatId).then(function (productKey) { var data = { "device": "lcd", "productKey": productKey, "action": "clear_message" } device.publish('anedongdong/' + productKey + "/lcd", JSON.stringify(data)); pubsub.sendMessage(chatId, "Clearing Message", {}); }) } else { pubsub.sendMessage(chatId, 'Try these instead\n\n' + 'On Lights:\n/onlight\n\n' + 'Off Lights:\n/offlight\n\n' + 'Display Message: (32 char limit)\n/displaymessage <message>' + '\n\nClear Message:\n/clearmessage' + '\n\nTake Photo:\n/picture' + '\n\nTake a 8s Video:\n/video', {}); } } else { pubsub.sendMessage(chatId, 'Your Telegram account is not linked to any of our products\n /link <ProductKey> to link your account', {}); } }) } }); var device = awsIot.device({ keyPath: './certs/private.pem.key', certPath: './certs/certificate.pem.crt', caPath: './certs/rootca.pem', clientId: "anedingdong", host: config.host }); device.on('connect', function () { console.log('MQTT connected'); device.subscribe('anedingding/#'); }); device.on('message', function (topic, payload) { var payloadData = JSON.parse(payload) var productKey = payloadData.productKey; var action = payloadData.action; user.getTelegramId(productKey).then(function (chatId) { switch (action) { case "button_pressed": { //send telegram message and save to firebase var datetime = payloadData.data.datetime; user.buttonPressed(datetime, productKey).then(function () { let replyOptions = { reply_markup: { resize_keyboard: true, one_time_keyboard: true, keyboard: [ ['Take Video'], ['Display a Message'] ], }, }; pubsub.sendMessage(chatId, 'Someone has pressed your doorbell, what do you want to do?', replyOptions); }) break; } case "led_on": { user.updateLEDStatus("on", productKey).then(function () { }) pubsub.sendMessage(chatId, 'Lights are On', {}); break; } case "led_off": { user.updateLEDStatus("off", productKey).then(function () { }) pubsub.sendMessage(chatId, 'Lights are Off', {}); break; } case "message_display": { var datetime = payloadData.data.datetime; var message = payloadData.data.message; user.displayMessage(datetime, message, productKey).then(function () { }) pubsub.sendMessage(chatId, 'Message Displayed', {}); break; } case "picture_taken": { var picturePath = payloadData.data.picturePath; var datetime = payloadData.data.datetime; user.pictureTaken(datetime, picturePath, productKey).then(function () { telegram.sendPhoto(chatId, picturePath, { caption: "Photo taken" }).catch(function (error) { console.log(error) }); }) break; } case "video_taken": { var videoPath = payloadData.data.videoPath; var datetime = payloadData.data.datetime; user.videoTaken(datetime, videoPath, productKey).then(function () { telegram.sendDocument(chatId, videoPath, { caption: "Video taken" }).catch(function (error) { console.log(error) }); }) break; } case "message_clear": { pubsub.sendMessage(chatId, 'Message Cleared', {}); break; } } }) });.

- Create a javascript file user.js and paste the code inside the file This file contains the methods related to Fireabse users node var user = module.exports = {}; var path = require('path'); var firebase = require('firebase'); var moment = require('moment'); var product = require(path.resolve('./APIs/product.js')); var pubsub = require(path.resolve('./APIs/pubsub.js')); var rootRef = firebase.database().ref('aneSmartDoorbell'); var userRef = rootRef.child('users'); // Create an account for the user user.signup = function (useremail, userpassword, productKey) { return new Promise(function (resolve, reject) { product.checkProductKey(productKey) //check if product available .then(function (available) { if (available == true) { firebase.auth().createUserWithEmailAndPassword(useremail, userpassword) .then(function (result) { //uid, email, emailVerified result.sendEmailVerification() .then(function () { //send email to user var newUser = { "email": result.email, "productKey": productKey, "uuid": result.uid, "telegramChatId": "" } var userKey = userRef.push(newUser) .then(function (data) { product.linkProductToUser(productKey, data.key); resolve("Successfully Sign up"); }).catch(function (error) { console.log("Error: " + error) reject("Unable to add user"); }) }).catch(function (error) { //verification email sent fail reject("Verification Email not sent"); }); }).catch(function (error) { console.log(JSON.stringify(error)) reject(error.message); }); } else { reject(available); } }); }); } //end of signup() user.signin = function (useremail, userpassword) { return new Promise(function (resolve, reject) { firebase.auth().signInWithEmailAndPassword(useremail, userpassword) .then(function (result) { if (result) { userRef.once('value', function (snapshot) { snapshot.forEach(function (oneUser) { if (oneUser.val().email == useremail) { resolve({ message: "Successfully Signed in", userid: oneUser.key, productkey: oneUser.val().productKey }) } }) }) } }).catch(function (error) { reject(error.message); }); }) } // get telegram id of product user.getTelegramId = function (productKey) { return new Promise(function (resolve, reject) { var gotId = false; userRef.once('value', function (snapshot) { snapshot.forEach(function (oneUser) { if (oneUser.val().productKey == productKey) { resolve(oneUser.val().telegramChatId); gotId = true; } }) if (!gotId) { reject; } }) }) } // update the user telegram id user.updateTelegramId = function (productKey, telegramId) { return new Promise(function (resolve, reject) { var updated = false; userRef.once('value', function (snapshot) { snapshot.forEach(function (oneUser) { if (oneUser.val().productKey == productKey) { userRef.child(oneUser.key).child("telegramChatId").set(telegramId); pubsub.sendMessage(telegramId, "Successfully link Telegram Account to your Product", {}); updated = true; } }) updated ? resolve("Success") : resolve("Failed"); }) }) } // check telegram id link to any product user.checkTelegramId = function (telegramId) { return new Promise(function (resolve, reject) { var isLink = false; userRef.once('value', function (snapshot) { snapshot.forEach(function (oneUser) { if (oneUser.val().telegramChatId == telegramId) { isLink = true; resolve(isLink); } }) if (!isLink) { resolve(isLink); } }) }) } // get telegram id of product user.getProductKey = function (chatId) { return new Promise(function (resolve, reject) { var gotId = false; userRef.once('value', function (snapshot) { snapshot.forEach(function (oneUser) { if (oneUser.val().telegramChatId == chatId) { resolve(oneUser.val().productKey); gotId = true; } }) if (!gotId) { reject; } }) }) } // add record of button press event to firebase user.buttonPressed = function (datetime, productKey) { return new Promise(function (resolve, reject) { userRef.once('value', function (snapshot) { snapshot.forEach(function (oneUser) { if (oneUser.val().productKey == productKey) { var newRecord = { "datetime": datetime } userRef.child(oneUser.key).child("device").child("button").push(newRecord, function (error) { if (error) { console.log("Error: " + error) reject(); } else { resolve(); } }); } }) }) }) } // add record of picture taken event to firebase user.pictureTaken = function (datetime, picturePath, productKey) { return new Promise(function (resolve, reject) { userRef.once('value', function (snapshot) { snapshot.forEach(function (oneUser) { if (oneUser.val().productKey == productKey) { var newRecord = { "datetime": datetime, "picturePath": picturePath } userRef.child(oneUser.key).child("device").child("camera").child("picture").push(newRecord, function (error) { if (error) { console.log("Error: " + error) reject(); } else { resolve(); } }); } }) }) }) } // add record of picture taken event to firebase user.videoTaken = function (datetime, videoPath, productKey) { return new Promise(function (resolve, reject) { userRef.once('value', function (snapshot) { snapshot.forEach(function (oneUser) { if (oneUser.val().productKey == productKey) { var newRecord = { "datetime": datetime, "videoPath": videoPath } userRef.child(oneUser.key).child("device").child("camera").child("video").push(newRecord, function (error) { if (error) { console.log("Error: " + error) reject(); } else { resolve(); } }); } }) }) }) } // add record of display message event to firebase user.displayMessage = function (datetime, message, productKey) { return new Promise(function (resolve, reject) { userRef.once('value', function (snapshot) { snapshot.forEach(function (oneUser) { if (oneUser.val().productKey == productKey) { var newRecord = { "datetime": datetime, "message": message } userRef.child(oneUser.key).child("device").child("lcd").push(newRecord, function (error) { if (error) { console.log("Error: " + error) reject(); } else { resolve(); } }); } }) }) }) } // update the led status user.updateLEDStatus = function (status, productKey) { return new Promise(function (resolve, reject) { userRef.once('value', function (snapshot) { snapshot.forEach(function (oneUser) { if (oneUser.val().productKey == productKey) { userRef.child(oneUser.key).child("device").child("led").child("status").set(status); resolve("Updated LED status") } }) }) }) } // return list of past user photos user.getUserPhoto = function (userKey) { return new Promise(function (resolve, reject) { userRef.child(userKey).child("device").child("camera").child("picture").once('value', function (snapshot) { var photoList = []; snapshot.forEach(function (data) { var onePhoto = { "datetime": moment.unix(data.val().datetime).format("dddd, DD MMMM YYYY, h:mm:ss a"), "picturePath": data.val().picturePath } photoList.push(onePhoto); }); resolve(photoList) }) }) } // return list of past user video downloadable links user.getUserVideo = function (userKey) { return new Promise(function (resolve, reject) { userRef.child(userKey).child("device").child("camera").child("video").once('value', function (snapshot) { var videoList = []; snapshot.forEach(function (data) { var oneVideo = { "datetime": moment.unix(data.val().datetime).format("dddd, DD MMMM YYYY, h:mm:ss a"), "videoPath": data.val().videoPath } videoList.push(oneVideo); }); resolve(videoList) }) }) } // return list of user button press history user.getUserButton = function (userKey) { return new Promise(function (resolve, reject) { userRef.child(userKey).child("device").child("button").once('value', function (snapshot) { var buttonList = []; var currentDate, newGrp; var existingGrp = false; var count = 0; snapshot.forEach(function (data) { var date = moment.unix(data.val().datetime).format("DD MMM YYYY"); if (currentDate != date) { //different date as before if (existingGrp) { //second different date newGrp = { "date": currentDate, "count": count } buttonList.push(newGrp) existingGrp = false; count = 1; } else { //first different date currentDate = date; existingGrp = true; newGrp = "" count += 1; } } else if (currentDate == date) { count += 1; } }); newGrp = { "date": currentDate, "count": count } buttonList.push(newGrp) resolve(buttonList) }) }) } // return list of user display message history user.getUserMessages = function (userKey) { return new Promise(function (resolve, reject) { userRef.child(userKey).child("device").child("lcd").once('value', function (snapshot) { var messageList = []; snapshot.forEach(function (data) { var oneMessage = { "datetime": moment.unix(data.val().datetime).format("dddd, DD MMMM YYYY, h:mm:ss a"), "message": data.val().message } messageList.push(oneMessage); }); resolve(messageList) }) }) } // return status of led user.getUserLight = function (userKey) { return new Promise(function (resolve, reject) { userRef.child(userKey).child("device").child("led").child("status").once('value', function (snapshot) { resolve(snapshot) }) }) }.

Task

- Create a javascript file config.js in the project main directory . ¦ └── config.js.

- Add the code below into your config.js file var config = module.exports = {} // AWS config.aws = { host: "", // AWS IoT Thing ARN (refer to Figure 1) port: 8883, } // Firebase Credentials config.firebaseCredentials = { // firebase console credentials } // Telegram Bot Id config.telegram = { token: "" // telegram bot token } // Gmail Credentials config.gmail = { 'email': '', // a gmail account 'password': '' // gmail account password }.

Figure 1:

Folder Checklist

. ¦ ├── .git ¦ ├── APIs ¦ ├── email.js ¦ ├── product.js ¦ ├── pubsub.js ¦ └── user.js ¦ ├── certs ¦ ├── certificate.pem.crt ¦ ├── private.pem.key ¦ ├── public.pem.key ¦ └── rootca.pem ¦ ├── Controllers ¦ ├── DeviceAPIController.js ¦ ├── ProductAPIController.js ¦ └── UserAPIController.js ¦ ├── node_modules ¦ ├── .cfignore ├── .gitignore ├── .project ├── app.js ├── config.js ├── LICENSE ├── manifest.yml ├── package.json └── README.md

- Telegram Bot

Task

At the end of this Section, you will have a Telegram Bot created and “linked” to your Node Server

Task

- In Telegram search for @BotFather

- /newbot to create a Telegram Bot - Give your bot a name and a username - After the bot is created copy the token as shown below

- Paste your token in the config.js file

- You can choose to set commands for the bot to help the user when they use the bot

For more information of other codes you can do with Telegram Bot: https://core.telegram.org/bots/api

- Firebase Console

Overview

At the end of this Section, you will have a Firebase Console app created and “linked” to the Node Server

Task

- Go to https://console.firebase.google.com/u/0/ and click on “Add project”

- Enter your Project Name and click Create Project

- Click on “Add Firebase to your web app”

- Copy the codes below

- Paste the code into the config.js file under Firebase Credentials

- To enable email and password authentication in Firebase, click on Authentication tab

- At the Sign-in method tab, click on the pencil icon at the right side of the Email/Password

- Click on Enable and then Save

- Change the Database rules of the Firebase app to allow the Node Server to read and write to Firebase { "rules": { ".read" : true, ".write" : true } }.

- Publish to Bluemix

Overview

In this section, we will publish the updated codes to Bluemix using Git Commands

Task

- Open command prompt in the Node Server project directory

- Execute the code git add * to add all the changes in the file

- git commit -m 'updated codes' to commit the changes added

- git push -u origin master to push the changes

- You can check your Bluemix Console to see whether it is updated

# Section 7
AnE Smart Doorbell Systems (Part 5)

- Web Application Setup

Overview

At the end of this section your folder directory should look like this . ¦ ├── certs ¦ ├── certificate.pem.crt ¦ ├── private.pem.key ¦ ├── public.pem.key ¦ └── rootca.pem ¦ ├── static ¦ └── css ¦ ├── login.css ¦ ├── register.css ¦ ├── styles.css ¦ └── welcome.css ¦ ├── templates ¦ ├── dashboard.html ¦ ├── lcd.html ¦ ├── light.html ¦ ├── login.html ¦ ├── picture.html ¦ ├── register.html ¦ └── welcome.html ¦ └── server.py

### Add AWS Certificates

Task

Add in your AWS Certs into your project directory for the Web application . ¦ └── certs ├── certificate.pem.crt ├── private.pem.key ├── public.pem.key └── rootca.pem

### server.py

Task

1. Create a new file named server.py in the project directory

2. Copy the following codes # coding: utf8 import gevent import gevent.monkey from gevent.pywsgi import WSGIServer from AWSIoTPythonSDK.MQTTLib import AWSIoTMQTTClient import json from datetime import datetime import calendar gevent.monkey.patch_all() from flask import Flask, request, Response, render_template, jsonify app = Flask(__name__) host = "" #specify your REST API Endpoint rootCAPath = "certs/rootca.pem" certificatePath = "certs/certificate.pem.crt" privateKeyPath = "certs/private.pem.key" my_rpi = AWSIoTMQTTClient("anesmartdoorbellweb") my_rpi.configureEndpoint(host, 8883) my_rpi.configureCredentials(rootCAPath, privateKeyPath, certificatePath) my_rpi.configureOfflinePublishQueueing(-1) # Infinite offline Publish queueing my_rpi.configureDrainingFrequency(2) # Draining: 2 Hz my_rpi.configureConnectDisconnectTimeout(10) # 10 sec my_rpi.configureMQTTOperationTimeout(5) # 5 sec my_rpi.connect() ########### Views ########### @app.route("/") def welcome(): return render_template('welcome.html') @app.route("/login") def login(): return render_template('login.html') @app.route("/register") def register(): return render_template('register.html') @app.route("/dashboard") def dashboard(): templateData = { 'title': 'Dashboard', 'main': 'AnE Smart Doorbell' } return render_template('dashboard.html', **templateData) @app.route("/picture") def picture(): templateData = { 'title': 'Pictures', 'main': 'AnE Smart Doorbell' } return render_template('picture.html', **templateData) @app.route("/light") def light(): templateData = { 'title': 'Lights', 'main': 'AnE Smart Doorbell' } return render_template('light.html', **templateData) @app.route("/message") def message(): templateData = { 'title': 'Display Message', 'main': 'AnE Smart Doorbell' } return render_template('lcd.html', **templateData) @app.route("/led", methods=["POST"]) def led(): data = json.loads(request.data) if (data["status"] == "on"): payload=json.dumps({ "device": "led", "productKey": data["productKey"], "action": "on_led" }) my_rpi.publish("anedongdong/" + data["productKey"] + "/led", payload, 1) elif (data["status"] == "off"): payload=json.dumps({ "device": "led", "productKey": data["productKey"], "action": "off_led" }) my_rpi.publish("anedongdong/" + data["productKey"] + "/led", payload, 1) return "done" @app.route("/camera", methods=["POST"]) def camera(): data = json.loads(request.data) if (data["action"] == "picture"): payload=json.dumps({ "device": "camera", "productKey": data["productKey"], "action": "take_picture" }) my_rpi.publish("anedongdong/" + data["productKey"] + "/camera", payload, 1) elif (data["action"] == "video"): payload=json.dumps({ "device": "camera", "productKey": data["productKey"], "action": "take_video" }) my_rpi.publish("anedongdong/" + data["productKey"] + "/camera", payload, 1) return "done" @app.route("/lcd", methods=["POST"]) def lcd(): data = json.loads(request.data) if (data["action"] == "clear"): payload=json.dumps({ "device": "lcd", "productKey": data["productKey"], "action": "clear_message" }) my_rpi.publish("anedongdong/" + data["productKey"] + "/lcd", payload, 1) elif (data["action"] == "display"): d = datetime.utcnow() timestamp = calendar.timegm(d.utctimetuple()) payload=json.dumps({ "device": "lcd", "productKey": data["productKey"], "action": "display_message", "data": { "message": data["message"], "datetime": timestamp } }) my_rpi.publish("anedongdong/" + data["productKey"] + "/lcd", payload, 1) return "done" if __name__ == '__main__': try: http_server = WSGIServer(('0.0.0.0', 8001), app) app.debug = True print("Web page running on port 8001") http_server.serve_forever() except: print("Exception")

Task

1. Create a new folder named templates to hold the html files in your project diectory . ¦ ├── certs ¦ ├── certificate.pem.crt ¦ ├── private.pem.key ¦ ├── public.pem.key ¦ └── rootca.pem ¦ ├──templates ¦ └── server.py

### welcome.html

Task

- Create file the file welcome.html in the templates folder and copy the following codes: <!DOCTYPE html> <html lang="en"> <head> <!-- Theme Made By www.w3schools.com - No Copyright --> <title>Bootstrap Theme The Band</title> <meta charset="utf-8"> <meta name="viewport" content="width=device-width, initial-scale=1"> <link rel="stylesheet" href="https://maxcdn.bootstrapcdn.com/bootstrap/3.3.7/css/bootstrap.min.css"> <link href="https://fonts.googleapis.com/css?family=Lato" rel="stylesheet" type="text/css"> <link href="https://fonts.googleapis.com/css?family=Montserrat" rel="stylesheet" type="text/css"> <script src="https://ajax.googleapis.com/ajax/libs/jquery/3.2.1/jquery.min.js"></script> <script src="https://maxcdn.bootstrapcdn.com/bootstrap/3.3.7/js/bootstrap.min.js"></script> <link rel="stylesheet" href="{{ url_for('static', filename='css/welcome.css') }}"> <style> </style> </head> <body id="myPage" data-spy="scroll" data-target=".navbar" data-offset="50"> <nav class="navbar navbar-default navbar-fixed-top"> <div class="container-fluid"> <div class="navbar-header"> <button type="button" class="navbar-toggle" data-toggle="collapse" data-target="#myNavbar"> <span class="icon-bar"></span> <span class="icon-bar"></span> <span class="icon-bar"></span> </button> <a class="navbar-brand" href="/">AnE Smart Door Bell </a> </div> <div class="collapse navbar-collapse" id="myNavbar"> <ul class="nav navbar-nav navbar-right"> <li><a href="login">LOGIN</a></li> <li><a href="register">REGISTER</a></li> </ul> </div> </div> </nav> <div id="myCarousel" class="carousel slide" data-ride="carousel"> <!-- Container (The Band Section) --> <div id="band" class="container text-center"> <h3>AnE SMART DOOR BELL SYSTEM</h3> <p><em>Notify you when someone press your doorbell!</em></p> <p>Telegram Bot</p> <br> </div> <!-- Container (Contact Section) --> <!-- Add Google Maps --> <div id="googleMap"></div> <script> function myMap() { var myCenter = new google.maps.LatLng(1.3083, 103.7797); var mapProp = { center: myCenter, zoom: 16, scrollwheel: false, draggable: false, mapTypeId: google.maps.MapTypeId.ROADMAP }; var map = new google.maps.Map(document.getElementById("googleMap"), mapProp); var marker = new google.maps.Marker({ position: myCenter }); marker.setMap(map); var map = new google.maps.Map(document.getElementById("googleMap"), mapProp); } </script> <script src="https://maps.googleapis.com/maps/api/js?key=AIzaSyCbkMdOajBjNodVhXnGW_LZ874WqIY54ws&callback=myMap"></script> <!-- Footer --> <footer class="text-center"> <a class="up-arrow" href="#myPage" data-toggle="tooltip" title="TO TOP"> <span class="glyphicon glyphicon-chevron-up"></span> </a> <br><br> <p>IoT Assignment CA2</p> </footer> <script> $(document).ready(function () { // Initialize Tooltip $('[data-toggle="tooltip"]').tooltip(); // Add smooth scrolling to all links in navbar + footer link $(".navbar a, footer a[href='#myPage']").on('click', function (event) { // Make sure this.hash has a value before overriding default behavior if (this.hash !== "") { // Prevent default anchor click behavior event.preventDefault(); // Store hash var hash = this.hash; // Using jQuery's animate() method to add smooth page scroll // The optional number (900) specifies the number of milliseconds it takes to scroll to the specified area $('html, body').animate({ scrollTop: $(hash).offset().top }, 900, function () { // Add hash (#) to URL when done scrolling (default click behavior) window.location.hash = hash; }); } // End if }); }) </script> </body> </html>

### login.html

Task

- Create file the file login.html in the templates folder and copy the following codes: - Replace the baseUrl with your Bluemix Node.js App url <!-- All the files that are required --> <link rel="stylesheet" href="https://maxcdn.bootstrapcdn.com/bootstrap/4.0.0-alpha.6/css/bootstrap.min.css" integrity="sha384-rwoIResjU2yc3z8GV/NPeZWAv56rSmLldC3R/AZzGRnGxQQKnKkoFVhFQhNUwEyJ" crossorigin="anonymous"> <link href='http://fonts.googleapis.com/css?family=Varela+Round' rel='stylesheet' type='text/css'> <script src="https://cdnjs.cloudflare.com/ajax/libs/limonte-sweetalert2/6.6.7/sweetalert2.min.js"></script> <link rel="stylesheet" href="https://cdnjs.cloudflare.com/ajax/libs/limonte-sweetalert2/6.6.7/sweetalert2.css" /> <script src="https://ajax.googleapis.com/ajax/libs/jquery/3.2.1/jquery.min.js"></script> <meta name="viewport" content="width=device-width, initial-scale=1, maximum-scale=1" /> <link rel="stylesheet" href="{{ url_for('static', filename='css/login.css') }}"> <!-- Main Form --> <div class="login-form-1"> <div class="logo">Login</div> <form id="login-form" class="text-left"> <div class="login-form-main-message"></div> <div class="main-login-form"> <div class="login-group"> <div class="form-group"> <label for="txtEmail" class="sr-only">Username</label> <input type="text" class="form-control" id="txtEmail" name="txtEmail" placeholder="Email"> </div> <div class="form-group"> <label for="txtPassword" class="sr-only">Password</label> <input type="password" class="form-control" id="txtPassword" name="txtPassword" placeholder="Password"> </div> </div> <button id="btnLogin" class=".login-button btn btn-primary" type="button">Login</i></button> </div> <div class="etc-login-form"> <p>Don't have an account? <a href="register">Register</a></p> </div> </form> </div> <script type="text/javascript"> (function () { var baseUrl = "https://anesmartdoorbell.au-syd.mybluemix.net"; var userKey = localStorage.getItem("userKey") var productKey = localStorage.getItem("productKey") if (userKey != null && productKey != null) { window.location.href = "dashboard" } $('#btnLogin').click(function () { var emailInput = $('#txtEmail').val() var passwordInput = $('#txtPassword').val() swal({ title: 'Logging in', showConfirmButton: false, imageUrl: "https://media.tenor.com/images/d6cd5151c04765d1992edfde14483068/tenor.gif", imageWidth: 80, imageHeight: 80 }); $.ajax({ url: baseUrl + '/api/user/signin', method: 'POST', data: { "email": emailInput, "password": passwordInput }, success: function (response) { if (response.success) { localStorage.setItem("userKey", response.userKey) localStorage.setItem("productKey", response.productKey) swal({ title: "Success", text: response.message, confirmButtonText: "Ok", type: "success" }).then(function () { window.location.href = "dashboard"; }); } else { $('#txtEmail').val(null) $('#txtPassword').val(null) swal("Error", response.message, "error"); } }, error: function (error) { console.dir(error) } }); }); })(); </script> <style> body { background: #3b3b3b; padding: 10px; font-family: 'Varela Round'; } </style>

### register.html

Task

- Create register.html in the templates folder and copy the following codes: - Replace the baseUrl with your Bluemix Node.js App url <!-- All the files that are required --> <link rel="stylesheet" href="https://maxcdn.bootstrapcdn.com/bootstrap/4.0.0-alpha.6/css/bootstrap.min.css" integrity="sha384-rwoIResjU2yc3z8GV/NPeZWAv56rSmLldC3R/AZzGRnGxQQKnKkoFVhFQhNUwEyJ" crossorigin="anonymous"> <link href='http://fonts.googleapis.com/css?family=Varela+Round' rel='stylesheet' type='text/css'> <script src="https://ajax.googleapis.com/ajax/libs/jquery/3.2.1/jquery.min.js"></script> <script src="https://cdnjs.cloudflare.com/ajax/libs/limonte-sweetalert2/6.6.7/sweetalert2.min.js"></script> <link rel="stylesheet" href="https://cdnjs.cloudflare.com/ajax/libs/limonte-sweetalert2/6.6.7/sweetalert2.css" /> <meta name="viewport" content="width=device-width, initial-scale=1, maximum-scale=1" /> <link rel="stylesheet" href="{{ url_for('static', filename='css/register.css') }}"> <!-- Main Form --> <div class="login-form-1"> <div class="logo">Register</div> <form id="login-form" class="text-left"> <div class="login-form-main-message"></div> <div class="main-login-form"> <div class="login-group"> <div class="form-group"> <label for="txtEmail" class="sr-only">Username</label> <input type="text" class="form-control" id="txtEmail" name="txtEmail" placeholder="Email"> </div> <div class="form-group"> <label for="txtPassword" class="sr-only">Password</label> <input type="password" class="form-control" id="txtPassword" name="txtPassword" placeholder="Password"> </div> <div class="form-group"> <label for="txtProductKey" class="sr-only">Product Key</label> <input type="text" class="form-control" id="txtProductKey" name="txtProductKey" placeholder="Product Key"> </div> </div> <button id="btnRegister" class=".login-button btn btn-primary" type="button">Register</i></button> </div> <div class="etc-login-form"> <p>Already have an account? <a href="login">Login</a></p> </div> </form> </div> <script type="text/javascript"> (function () { var baseUrl = "https://anesmartdoorbell.au-syd.mybluemix.net"; var userKey = localStorage.getItem("userKey") var productKey = localStorage.getItem("productKey") if (userKey != null && productKey != null) { window.location.href = "dashboard" } $('#btnRegister').click(function () { console.log("ahah") var emailInput = $('#txtEmail').val() var passwordInput = $('#txtPassword').val() var productKeyInput = $('#txtProductKey').val() $.ajax({ url: baseUrl + '/api/user/signup', method: 'POST', data: { "email": emailInput, "password": passwordInput, "productKey": productKeyInput }, success: function (response) { if (response.success) { swal("Success", response.message, "success"); window.location.href = "login" } else { swal("Error", response.message, "error"); } }, error: function (error) { console.dir(error) } }); }); })(); </script>

### dashboard.html

Task

- Create dashboard.html in the templates folder and copy the following codes: - Replace the baseUrl with your Bluemix Node.js App url <head> <title>{{ title }}</title> <script src="https://ajax.googleapis.com/ajax/libs/jquery/3.2.1/jquery.min.js"></script> <script src="https://gitcdn.github.io/bootstrap-toggle/2.2.2/js/bootstrap-toggle.min.js"></script> <script src="http://d3js.org/d3.v3.min.js"></script> <script src="http://dimplejs.org/dist/dimple.v2.1.6.min.js"></script> <script src="https://cdnjs.cloudflare.com/ajax/libs/limonte-sweetalert2/6.6.7/sweetalert2.min.js"></script> <link rel="stylesheet" href="https://cdnjs.cloudflare.com/ajax/libs/limonte-sweetalert2/6.6.7/sweetalert2.css" /> <link href="https://gitcdn.github.io/bootstrap-toggle/2.2.2/css/bootstrap-toggle.min.css" rel="stylesheet"> <link rel="stylesheet" href="https://maxcdn.bootstrapcdn.com/bootstrap/3.3.7/css/bootstrap.min.css" /> <link rel="stylesheet" href="{{ url_for('static', filename='css/styles.css') }}"> </head> <body> <div class="container-fluid"> <div class="row content"> <div class="col-sm-3 sidenav hidden-xs"> <h3>{{main}}</h3> <p> Welcome to AnE smart systems... </p> <hr/> <div> <h3 id="Time"></h3> <h4 id="Date"></h4> </div> <hr /> <ul class="nav navbar-collapse nav-stacked" id="navigations"> <li class="active"><a href="dashboard"><span class="glyphicon glyphicon-home"></span> Dashboard</a></li> <li><a href="picture"><span class="glyphicon glyphicon-picture"></span> View Pictures</a></li> <li><a href="light"><span class="glyphicon glyphicon-lamp"></span> Lights</a></li> <li><a href="message"><span class="glyphicon glyphicon-comment"></span> Messages</a></li> <li><a href="/" id="logout"><span class="glyphicon glyphicon-log-out"></span> Logout</a></li> </ul> <br> </div> <br> <div class="col-sm-9"> <h2 class="text-center">{{title}}</h2> <hr /> <div class="panel panel-default"> <div class="panel-heading custom_class"> <h3 class="panel-title"> Number of times doorbell is pressed by Days: </h3> </div> <div class="panel-body"> <div id="chartContainer"></div> </div> <div class="panel-footer"> <span class="glyphicon glyphicon-time"></span> <p id="lastUpdate"></p> </div> </div> </div> </div> </div> </body> <script> (function () { /************************** Variables *************************/ var baseUrl = "https://anesmartdoorbell.au-syd.mybluemix.net/" var userKey = localStorage.getItem("userKey") var svg var chart var productKey = localStorage.getItem('productKey') if (userKey == null ¦¦ productKey == null) { swal({ title: "Error", text: "You are not Login", confirmButtonText: "Login here", type: "error" }).then(function () { localStorage.removeItem('userKey') localStorage.removeItem('productKey') window.location.href = "login"; }); } /************************* Functions *************************/ //############################################### //--------------- checkTime()------------------// // add zero in front of numbers < 10 //############################################### function checkTime(i) { if (i < 10) { i = "0" + i }; return i; } //############################################### //--------------- startTime()------------------// // start running the clock on the web page //############################################### function startTime() { var now = new Date; var map = { '13': 1, '14': 2, '15': 3, '16': 4, '17': 5, '18': 6, '19': 7, '20': 8, '21': 9, '22': 10, '23': 11, '00': 12 } h = checkTime(now.getHours()); m = checkTime(now.getMinutes()); s = checkTime(now.getSeconds()); var ampm = (h >= 12) ? "PM" : "AM"; var hh = (h > 12 ¦¦ h == 00) ? map[h] : h var time = hh + ':' + m + ':' + s + ' ' + ampm; document.getElementById('Time').innerHTML = time } //########################################## //--------------- date()------------------// // display the date on the web page //########################################## function date() { var now = new Date; year = now.getFullYear(); month = now.getMonth(); months = new Array('January', 'February', 'March', 'April', 'May', 'June', 'Jully', 'August', 'September', 'October', 'November', 'December'); d = now.getDate(); day = now.getDay(); days = new Array('Sunday', 'Monday', 'Tuesday', 'Wednesday', 'Thursday', 'Friday', 'Saturday'); date = '' + days[day] + ', ' + d + ' ' + months[month] + ' ' + year; $('#Date').text(date) } //########################################## //--------------- time()------------------// // returns formatted current time //########################################## function time() { var now = new Date; var map = { '13': 1, '14': 2, '15': 3, '16': 4, '17': 5, '18': 6, '19': 7, '20': 8, '21': 9, '22': 10, '23': 11, '00': 12 } h = checkTime(now.getHours()); m = checkTime(now.getMinutes()); s = checkTime(now.getSeconds()); var ampm = (h >= 12) ? "PM" : "AM"; var hh = (h > 12 ¦¦ h == 00) ? map[h] : h var time = hh + ':' + m + ':' + s + ' ' + ampm; return time; } //##################################################### //--------------- onRefreshChart()------------------// // get latest light sensor data //##################################################### function onRefreshChart() { var now = new Date(); $('#lastUpdate').text("Last Updated: " + now.getDate() + "/" + (now.getMonth() + 1) + "/" + now.getFullYear() + " " + time()) $.ajax({ url: baseUrl + "api/device/button", method: "GET", headers: { "content-type": "application/json", "userkey": userKey }, success: function (result) { onGetResult(result.data) } }) } //################################################## //--------------- onGetResult()------------------// // loads data into chart //################################################## function onGetResult(result) { chart.data = result; chart.draw(500); } /************************* Run Functions *************************/ date(); setInterval(startTime, 500); svg = dimple.newSvg("#chartContainer", 800, 400); chart = new dimple.chart(svg, null); x = chart.addCategoryAxis("x", "date"); x.title = "Date"; y = chart.addMeasureAxis("y", "count"); y.title = "Number of times pressed"; series = chart.addSeries(null, dimple.plot.bar); onRefreshChart(); setInterval(onRefreshChart, 10000); /************************* Events *************************/ $('#logout').on('click', function () { swal({ title: 'Logging out', showConfirmButton: false, }); localStorage.removeItem("userKey"); localStorage.removeItem("productKey"); }) }()) </script>

### lcd.html

Task

- Create lcd.html in the templates folder and copy the following codes: - Replace the baseUrl with your Bluemix Node.js App url <head> <title>{{ title }}</title> <script src="https://ajax.googleapis.com/ajax/libs/jquery/3.2.1/jquery.min.js"></script> <script src="https://gitcdn.github.io/bootstrap-toggle/2.2.2/js/bootstrap-toggle.min.js"></script> <script src="https://cdnjs.cloudflare.com/ajax/libs/limonte-sweetalert2/6.6.7/sweetalert2.min.js"></script> <link rel="stylesheet" href="https://cdnjs.cloudflare.com/ajax/libs/limonte-sweetalert2/6.6.7/sweetalert2.css" /> <link href="https://gitcdn.github.io/bootstrap-toggle/2.2.2/css/bootstrap-toggle.min.css" rel="stylesheet"> <link rel="stylesheet" href="https://maxcdn.bootstrapcdn.com/bootstrap/3.3.7/css/bootstrap.min.css" /> <link rel="stylesheet" href="{{ url_for('static', filename='css/styles.css') }}"> </head> <body> <div class="container-fluid"> <div class="row content"> <div class="col-sm-3 sidenav hidden-xs"> <h3>{{main}}</h3> <p> Welcome to AnE smart systems... </p> <hr /> <div> <h3 id="Time"></h3> <h4 id="Date"></h4> </div> <hr /> <ul class="nav navbar-collapse nav-stacked" id="navigations"> <li class="active"><a href="dashboard"><span class="glyphicon glyphicon-home"></span> Dashboard</a></li> <li><a href="picture"><span class="glyphicon glyphicon-picture"></span> View Pictures</a></li> <li><a href="light"><span class="glyphicon glyphicon-lamp"></span> Lights</a></li> <li><a href="message"><span class="glyphicon glyphicon-comment"></span> Messages</a></li> <li><a href="/" id="logout"><span class="glyphicon glyphicon-log-out"></span> Logout</a></li> </ul> <br> </div> <br> <div class="col-sm-9"> <h2 class="text-center">{{title}}</h2> <hr /> <div class="panel panel-default"> <div class="panel-heading custom_class"> <h3 class="panel-title"> Camera </h3> </div> <div class="panel-body"> <label for="messageInput">Display Message:</label> <input type="text" class="form-control" id="messageInput" maxlength="32"><br /> <button id="displayMessage" class="btn primary">Display Message on LCD</button> <button id="clearMessage" class="btn primary">Clear Message on LCD</button> </div> </div> <div class="panel panel-default"> <div class="panel-heading custom_class"> <h3 class="panel-title"> Message History </h3> </div> <div class="panel-body"> <table id="userMessage"></table> </div> <div class="panel-footer"> </div> </div> </div> </div> </div> </body> <script> (function () { /************************** Variables *************************/ var baseUrl = "https://anesmartdoorbell.au-syd.mybluemix.net"; var userKey = localStorage.getItem('userKey') var productKey = localStorage.getItem('productKey') if (userKey == null ¦¦ productKey == null) { swal({ title: "Error", text: "You are not Login", confirmButtonText: "Login here", type: "error" }).then(function () { localStorage.removeItem('userKey') localStorage.removeItem('productKey') window.location.href = "login"; }); } /************************* Functions *************************/ function displayMsgHistory() { $.ajax({ // Post username, password & the grant type to /token url: baseUrl + '/api/device/message', method: 'GET', headers: { "content-type": "application/json", "userkey": userKey }, success: function (response) { messageList = response.data; var $tableElement = $('#userMessage'); var $dateTimeHeaderElement = $('<th></th>'); $dateTimeHeaderElement.addClass('col-md-4 col-sm-6') $dateTimeHeaderElement.text("DateTimeSent"); var $messageHeaderElement = $('<th></th>'); $messageHeaderElement.addClass('col-md-4 col-sm-6') $messageHeaderElement.text("Message"); var $headerRowElement = $('<tr></tr>'); $headerRowElement.append($dateTimeHeaderElement); $headerRowElement.append($messageHeaderElement); $tableElement.append($headerRowElement); messageList.forEach(function (oneMessage) { var messageDateTime = oneMessage.datetime var messageMsg = oneMessage.message; var $oneMessageRowElement = $('<tr></tr>'); var $dateTimeElement = $('<td></td>'); $dateTimeElement.addClass('col-md-4 col-sm-6') $dateTimeElement.text(messageDateTime); var $messageElement = $('<td></td>'); $messageElement.addClass('col-md-4 col-sm-6') $messageElement.text(messageMsg); $oneMessageRowElement.append($dateTimeElement); $oneMessageRowElement.append($messageElement); $tableElement.append($oneMessageRowElement) }); }, error: function (error) { console.dir(error) } }); } //############################################### //--------------- checkTime()------------------// // add zero in front of numbers < 10 //############################################### function checkTime(i) { if (i < 10) { i = "0" + i }; return i; } //############################################### //--------------- startTime()------------------// // start running the clock on the web page //############################################### function startTime() { var now = new Date; var map = { '13': 1, '14': 2, '15': 3, '16': 4, '17': 5, '18': 6, '19': 7, '20': 8, '21': 9, '22': 10, '23': 11, '00': 12 } h = checkTime(now.getHours()); m = checkTime(now.getMinutes()); s = checkTime(now.getSeconds()); var ampm = (h >= 12) ? "PM" : "AM"; var hh = (h > 12 ¦¦ h == 00) ? map[h] : h var time = hh + ':' + m + ':' + s + ' ' + ampm; document.getElementById('Time').innerHTML = time var t = setTimeout(startTime, 500); } //########################################## //--------------- date()------------------// // display the date on the web page //########################################## function date() { var now = new Date; year = now.getFullYear(); month = now.getMonth(); months = new Array('January', 'February', 'March', 'April', 'May', 'June', 'Jully', 'August', 'September', 'October', 'November', 'December'); d = now.getDate(); day = now.getDay(); days = new Array('Sunday', 'Monday', 'Tuesday', 'Wednesday', 'Thursday', 'Friday', 'Saturday'); date = '' + days[day] + ', ' + d + ' ' + months[month] + ' ' + year; $('#Date').text(date) } $('#displayMessage').on('click', function () { $.ajax({ url: "/lcd", type: "POST", contentType: "application/json", data: JSON.stringify({ "action": "display", "productKey": productKey, "message": $('#messageInput').val() }), success: function (response) { } }) }) $('#clearMessage').on('click', function () { $.ajax({ url: "/lcd", type: "POST", contentType: "application/json", data: JSON.stringify({ "action": "clear", "productKey": productKey }), success: function (response) { } }) }) $('#logout').on('click', function () { swal({ title: 'Logging out', showConfirmButton: false, }); localStorage.removeItem("userKey"); localStorage.removeItem("productKey"); }) /************************* Run Functions *************************/ date(); setInterval(startTime, 500) displayMsgHistory() }()) </script> <style> #userMessage { width: 90%; margin: auto; } table, th, td { border: 1px solid black; border-collapse: collapse; } </style>

### light.html

Task

- Create light.html in the templates folder and copy the following codes: - Replace the baseUrl with your Bluemix Node.js App url <!DOCTYPE html> <head> <title>{{ title }}</title> <script src="https://ajax.googleapis.com/ajax/libs/jquery/3.2.1/jquery.min.js"></script> <script src="https://gitcdn.github.io/bootstrap-toggle/2.2.2/js/bootstrap-toggle.min.js"></script> <script src="https://cdnjs.cloudflare.com/ajax/libs/limonte-sweetalert2/6.6.7/sweetalert2.min.js"></script> <link rel="stylesheet" href="https://cdnjs.cloudflare.com/ajax/libs/limonte-sweetalert2/6.6.7/sweetalert2.css" /> <link href="https://gitcdn.github.io/bootstrap-toggle/2.2.2/css/bootstrap-toggle.min.css" rel="stylesheet"> <link rel="stylesheet" href="https://maxcdn.bootstrapcdn.com/bootstrap/3.3.7/css/bootstrap.min.css" /> <link rel="stylesheet" href="{{ url_for('static', filename='css/styles.css') }}"> </head> <body> <div class="container-fluid"> <div class="row content"> <div class="col-sm-3 sidenav hidden-xs"> <h3>{{main}}</h3> <p> Welcome to AnE smart systems... </p> <hr /> <div> <h3 id="Time"></h3> <h4 id="Date"></h4> </div> <hr /> <ul class="nav navbar-collapse nav-stacked" id="navigations"> <li><a href="dashboard"><span class="glyphicon glyphicon-home"></span> Dashboard</a></li> <li><a href="picture"><span class="glyphicon glyphicon-picture"></span> View Pictures</a></li> <li class="active"><a href="light"><span class="glyphicon glyphicon-lamp"></span> Lights</a></li> <li><a href="message"><span class="glyphicon glyphicon-comment"></span> Messages</a></li> <li><a href="/" id="logout"><span class="glyphicon glyphicon-log-out"></span> Logout</a></li> </ul> <br> </div> <br> <div class="col-sm-9"> <h2 class="text-center">{{title}}</h2> <hr /> <div class="panel panel-default"> <div class="panel-heading custom_class"> <h3 class="panel-title"> Light Status </h3> </div> <div class="panel-body"> <section class="row text-center placeholders"> <div class="col-6 col-sm-3 placeholder"> <img src="https://media.licdn.com/mpr/mpr/AAEAAQAAAAAAAAdeAAAAJGZjMTBkYzRkLWYzZTgtNGE1YS1iOWQ2LTk0ZmRhNjZhNGQ1ZQ.png" width="80" height="80" class="img-fluid rounded-circle" alt="Generic placeholder thumbnail"> <h4>LED</h4> <span class="text-muted">Lights up when user wants to deter intruders.</span><br/><br/> <input id="ledStatus" type="checkbox" checked data-toggle="toggle"> </div> <!--<div class="col-6 col-sm-3 placeholder">--> <!-- <label id="switch" class="switch"> <input type="checkbox"> <span class="on">ON</span><span class="off">OFF</span> <span class="slider round"></span> </label> --> <!--</div>--> </section> </div> <div class="panel-footer"> </div> </div> </div> </div> </div> </body> </html> <script> (function () { /************************** Variables *************************/ var ledStatus = $('#ledStatus');; var baseUrl = "https://anesmartdoorbell.au-syd.mybluemix.net"; var userKey = localStorage.getItem('userKey') var productKey = localStorage.getItem('productKey') if (userKey == null ¦¦ productKey == null) { swal({ title: "Error", text: "You are not Login", confirmButtonText: "Login here", type: "error" }).then(function () { localStorage.removeItem('userKey') localStorage.removeItem('productKey') window.location.href = "login"; }); } /************************* Functions *************************/ function updateToggle() { $.ajax({ url: baseUrl + '/api/device/led', method: 'GET', headers: { "content-type": "application/json", "userkey": userKey }, success: function (response) { if (response.success) { var status = response.data == "on" ? true : false if (status) { if (status != $('#ledStatus').prop('checked')) { $('#ledStatus').bootstrapToggle('on') } } else { if (status != $('#ledStatus').prop('checked')) { $('#ledStatus').bootstrapToggle('off') } } } }, error: function (error) { console.dir(error) } }); } //############################################### //--------------- checkTime()------------------// // add zero in front of numbers < 10 //############################################### function checkTime(i) { if (i < 10) { i = "0" + i }; return i; } //############################################### //--------------- startTime()------------------// // start running the clock on the web page //############################################### function startTime() { var now = new Date; var map = { '13': 1, '14': 2, '15': 3, '16': 4, '17': 5, '18': 6, '19': 7, '20': 8, '21': 9, '22': 10, '23': 11, '00': 12 } h = checkTime(now.getHours()); m = checkTime(now.getMinutes()); s = checkTime(now.getSeconds()); var ampm = (h >= 12) ? "PM" : "AM"; var hh = (h > 12 ¦¦ h == 00) ? map[h] : h var time = hh + ':' + m + ':' + s + ' ' + ampm; document.getElementById('Time').innerHTML = time } //########################################## //--------------- date()------------------// // display the date on the web page //########################################## function date() { var now = new Date; year = now.getFullYear(); month = now.getMonth(); months = new Array('January', 'February', 'March', 'April', 'May', 'June', 'Jully', 'August', 'September', 'October', 'November', 'December'); d = now.getDate(); day = now.getDay(); days = new Array('Sunday', 'Monday', 'Tuesday', 'Wednesday', 'Thursday', 'Friday', 'Saturday'); date = '' + days[day] + ', ' + d + ' ' + months[month] + ' ' + year; $('#Date').text(date) } /************************* Run Functions *************************/ date(); setInterval(startTime, 500) setInterval(updateToggle, 10000) /************************* Events *************************/ var firstTime = true $('#ledStatus').on("change", function () { if (!firstTime) { var status = $(this).prop('checked') ? "on" : "off"; $.ajax({ url: "/led", type: "POST", contentType: "application/json", data: JSON.stringify({ "status": status, "productKey": productKey }), success: function (response) { } }) } else { firstTime = false } }); $('#logout').on('click', function () { swal({ title: 'Logging out', showConfirmButton: false, }); localStorage.removeItem("userKey"); localStorage.removeItem("productKey"); }) }()) </script> <style> .switch { position: relative; display: inline-block; width: 60px; height: 34px; } /* Hide default HTML checkbox */ .switch input { display: none; } /* The slider */ .slider { position: absolute; cursor: pointer; top: 0; left: 0; right: 0; bottom: 0; background-color: #ccc; -webkit-transition: .4s; transition: .4s; } .slider:before { position: absolute; content: ""; height: 26px; width: 26px; left: 4px; bottom: 4px; background-color: white; -webkit-transition: .4s; transition: .4s; } input:checked+.slider { background-color: #2196F3; } input:focus+.slider { box-shadow: 0 0 1px #2196F3; } input:checked+.slider:before { -webkit-transform: translateX(26px); -ms-transform: translateX(26px); transform: translateX(26px); } /* Rounded sliders */ .slider.round { border-radius: 34px; } .slider.round:before { border-radius: 50%; } .slider:after { content: 'OFF'; color: white; display: block; position: absolute; transform: translate(-50%, -50%); top: 50%; left: 50%; font-size: 10px; font-family: Verdana, sans-serif; } input:checked+.slider:after { content: 'ON'; } </style>

### picture.html

Task

- Create pictures.html in the templates folder and copy the following codes: - Replace the baseUrl with your Bluemix Node.js App url <head> <title>{{ title }}</title> <script src="https://ajax.googleapis.com/ajax/libs/jquery/3.2.1/jquery.min.js"></script> <script src="https://gitcdn.github.io/bootstrap-toggle/2.2.2/js/bootstrap-toggle.min.js"></script> <script src="https://cdnjs.cloudflare.com/ajax/libs/limonte-sweetalert2/6.6.7/sweetalert2.min.js"></script> <link rel="stylesheet" href="https://cdnjs.cloudflare.com/ajax/libs/limonte-sweetalert2/6.6.7/sweetalert2.css" /> <link href="https://gitcdn.github.io/bootstrap-toggle/2.2.2/css/bootstrap-toggle.min.css" rel="stylesheet"> <link rel="stylesheet" href="https://maxcdn.bootstrapcdn.com/bootstrap/3.3.7/css/bootstrap.min.css" /> <link rel="stylesheet" href="{{ url_for('static', filename='css/styles.css') }}"> </head> <body> <div class="container-fluid"> <div class="row content"> <div class="col-sm-3 sidenav hidden-xs"> <h3>{{main}}</h3> <p> Welcome to AnE smart systems... </p> <hr /> <div> <h3 id="Time"></h3> <h4 id="Date"></h4> </div> <hr /> <ul class="nav navbar-collapse nav-stacked" id="navigations"> <li><a href="dashboard"><span class="glyphicon glyphicon-home"></span> Dashboard</a></li> <li class="active"><a href="picture"><span class="glyphicon glyphicon-picture"></span> View Pictures</a></li> <li><a href="light"><span class="glyphicon glyphicon-lamp"></span> Lights</a></li> <li><a href="message"><span class="glyphicon glyphicon-comment"></span> Messages</a></li> <li><a href="/" id="logout"><span class="glyphicon glyphicon-log-out"></span> Logout</a></li> </ul> <br> </div> <br> <div class="col-sm-9"> <h2 class="text-center">{{title}}</h2> <hr /> <div class="panel panel-default"> <div class="panel-heading custom_class"> <h3 class="panel-title"> Camera </h3> </div> <div class="panel-body"> <button id="takePicture" class="btn primary">Take a Picture</button> <button id="takeVideo" class="btn primary">Take a Video</button> </div> </div> <div class="col-lg-12"> <h3>Pictures Taken</h3> <hr> </div> <div class="row text-center" id="userImg"> </div> <div class="col-lg-12"> <h3>Videos Taken</h3> <hr> </div> <div class="row text-center" id="userVideo"> </div> </div> </div> </div> </body> <script> (function () { /************************** Variables *************************/ var baseUrl = "https://anesmartdoorbell.au-syd.mybluemix.net/" var userKey = localStorage.getItem("userKey") var productKey = localStorage.getItem('productKey') if (userKey == null ¦¦ productKey == null) { swal({ title: "Error", text: "You are not Login", confirmButtonText: "Login here", type: "error" }).then(function () { localStorage.removeItem('userKey') localStorage.removeItem('productKey') window.location.href = "login"; }); } /************************* Functions *************************/ //############################################### //--------------- checkTime()------------------// // add zero in front of numbers < 10 //############################################### function checkTime(i) { if (i < 10) { i = "0" + i }; return i; } //############################################### //--------------- startTime()------------------// // start running the clock on the web page //############################################### function startTime() { var now = new Date; var map = { '13': 1, '14': 2, '15': 3, '16': 4, '17': 5, '18': 6, '19': 7, '20': 8, '21': 9, '22': 10, '23': 11, '00': 12 } h = checkTime(now.getHours()); m = checkTime(now.getMinutes()); s = checkTime(now.getSeconds()); var ampm = (h >= 12) ? "PM" : "AM"; var hh = (h > 12 ¦¦ h == 00) ? map[h] : h var time = hh + ':' + m + ':' + s + ' ' + ampm; document.getElementById('Time').innerHTML = time } //########################################## //--------------- date()------------------// // display the date on the web page //########################################## function date() { var now = new Date; year = now.getFullYear(); month = now.getMonth(); months = new Array('January', 'February', 'March', 'April', 'May', 'June', 'Jully', 'August', 'September', 'October', 'November', 'December'); d = now.getDate(); day = now.getDay(); days = new Array('Sunday', 'Monday', 'Tuesday', 'Wednesday', 'Thursday', 'Friday', 'Saturday'); date = '' + days[day] + ', ' + d + ' ' + months[month] + ' ' + year; $('#Date').text(date) } //########################################## //------------ listPicture()---------------// // display pictures //########################################## function listPicture() { $('#userImg').empty() $.ajax({ "url": baseUrl + "api/device/photo", "method": "GET", "headers": { "content-type": "application/json", "userkey": userKey }, success: function (response) { imgList = response.data imgList.forEach(function (oneImg) { var imgDescription = oneImg.datetime; var $photoClickLink = $('<a>').attr("href", "#"); var $container = $('<div>').addClass('col-md-4 col-sm-6'); var $imageContainer = $('<div>').addClass('thumbnail'); var $image = $('<img>').attr('src', oneImg.picturePath); var $captionContainer = $('<div>').addClass('caption'); var $caption = $('<p>').text(imgDescription); var $containerImg = $imageContainer.append($image); var $containerCaption = $captionContainer.append($caption); var $containerThumbnail = $imageContainer.append($containerImg).append($containerCaption); var $parentContainer = $container.append($containerThumbnail); var $finalContainer = $photoClickLink.append($parentContainer); $('#userImg').append($finalContainer); }); } }); } //########################################## //------------ listVideo()---------------// // display videos //########################################## function listVideo() { $('#userVideo').empty() $.ajax({ "url": baseUrl + "api/device/video", "method": "GET", "headers": { "content-type": "application/json", "userkey": userKey }, success: function (response) { response.data.forEach(function (oneVid) { var vidDescription = oneVid.datetime; var $container = $('<div>').addClass('col-md-6 col-sm-6 video'); var $video = $('<video width="320" height="240" controls>'); var $videoSrc = $('<source>').attr('src', oneVid.videoPath); var $videoContainer = $video.append($videoSrc); var $captionContainer = $('<div>').addClass('caption'); var $caption = $('<p>').text(vidDescription); var $containerCaption = $captionContainer.append($caption); var $parentContainer = $container.append($videoContainer).append($containerCaption); $('#userVideo').append($parentContainer); }); } }); } /************************* Run Functions *************************/ date(); listPicture(); listVideo(); setInterval(startTime, 500) setInterval(listPicture, 60000) setInterval(listVideo, 60000) /************************* Events *************************/ $('#takePicture').on('click', function () { $.ajax({ url: "/camera", type: "POST", contentType: "application/json", data: JSON.stringify({ "action": "picture", "productKey": productKey }), success: function (response) { } }) }) $('#takeVideo').on('click', function () { $.ajax({ url: "/camera", type: "POST", contentType: "application/json", data: JSON.stringify({ "action": "video", "productKey": productKey }), success: function (response) { } }) }) $('#logout').on('click', function () { swal({ title: 'Logging out', showConfirmButton: false, }); localStorage.removeItem("userKey"); localStorage.removeItem("productKey"); }) }()) </script>

Setup static folder

1. Create a new folder named static/css . ¦ ├── certs ¦ ├── certificate.pem.crt ¦ ├── private.pem.key ¦ ├── public.pem.key ¦ └── rootca.pem ¦ ├── static ¦ └── css ¦ ├── templates ¦ ├── dashboard.html ¦ ├── lcd.html ¦ ├── light.html ¦ ├── login.html ¦ ├── picture.html ¦ ├── register.html ¦ └── welcome.html ¦ └── server.py

### welcome.css

welcome.css

1. Create welcome.css in the static/css folder and copy the following codes: body { font: 400 15px/1.8 Lato, sans-serif; color: #777; } h3, h4 { margin: 10px 0 30px 0; letter-spacing: 10px; font-size: 20px; color: #111; } .container { padding: 80px 120px; } .person { border: 10px solid transparent; margin-bottom: 25px; width: 80%; height: 80%; opacity: 0.7; } .person:hover { border-color: #f1f1f1; } .carousel-inner img { -webkit-filter: grayscale(90%); filter: grayscale(90%); /* make all photos black and white */ width: 100%; /* Set width to 100% */ margin: auto; } .carousel-caption h3 { color: #fff !important; } @media (max-width: 600px) { .carousel-caption { display: none; /* Hide the carousel text when the screen is less than 600 pixels wide */ } } .bg-1 { background: #2d2d30; color: #bdbdbd; } .bg-1 h3 { color: #fff; } .bg-1 p { font-style: italic; } .list-group-item:first-child { border-top-right-radius: 0; border-top-left-radius: 0; } .list-group-item:last-child { border-bottom-right-radius: 0; border-bottom-left-radius: 0; } .thumbnail { padding: 0 0 15px 0; border: none; border-radius: 0; } .thumbnail p { margin-top: 15px; color: #555; } .btn { padding: 10px 20px; background-color: #333; color: #f1f1f1; border-radius: 0; transition: .2s; } .btn:hover, .btn:focus { border: 1px solid #333; background-color: #fff; color: #000; } .modal-header, h4, .close { background-color: #333; color: #fff !important; text-align: center; font-size: 30px; } .modal-header, .modal-body { padding: 40px 50px; } .nav-tabs li a { color: #777; } #googleMap { width: 100%; height: 400px; -webkit-filter: grayscale(100%); filter: grayscale(100%); } .navbar { font-family: Montserrat, sans-serif; margin-bottom: 0; background-color: #2d2d30; border: 0; font-size: 11px !important; letter-spacing: 4px; opacity: 0.9; } .navbar li a, .navbar .navbar-brand { color: #d5d5d5 !important; } .navbar-nav li a:hover { color: #fff !important; } .navbar-nav li.active a { color: #fff !important; background-color: #29292c !important; } .navbar-default .navbar-toggle { border-color: transparent; } .open .dropdown-toggle { color: #fff; background-color: #555 !important; } .dropdown-menu li a { color: #000 !important; } .dropdown-menu li a:hover { background-color: #555 !important; } footer { background-color: #2d2d30; color: #f5f5f5; padding: 32px; } footer a { color: #f5f5f5; } footer a:hover { color: #777; text-decoration: none; } .form-control { border-radius: 0; } textarea { resize: none; }

### login.css

login.css

1. Create login.css in the static/css folder and copy the following codes: a { color: #aaaaaa; transition: all ease-in-out 200ms; } a:hover { color: #333333; text-decoration: none; } /*=== 3. Text Outside the Box ===*/ .etc-login-form { color: #919191; padding: 10px 20px; clear: right; } .etc-login-form p { margin-bottom: 5px; } /*=== 4. Main Form ===*/ .login-form-1 { max-width: 300px; border-radius: 5px; /*display: inline-block;*/ margin: auto; } .main-login-form { position: relative; } .login-form-1 .form-control { border: 0; box-shadow: 0 0 0; border-radius: 0; background: transparent; color: #555555; padding: 7px 0; font-weight: bold; height: auto; } .login-form-1 .form-control::-webkit-input-placeholder { color: #999999; } .login-form-1 .form-control:-moz-placeholder, .login-form-1 .form-control::-moz-placeholder, .login-form-1 .form-control:-ms-input-placeholder { color: #999999; } .login-form-1 .form-group { margin-bottom: 0; border-bottom: 2px solid #efefef; padding-right: 20px; position: relative; } .login-form-1 .form-group:last-child { border-bottom: 0; } .login-group { background: #ffffff; color: #999999; border-radius: 8px; padding: 10px 20px; margin-bottom: 5%; } .login-group-checkbox { padding: 5px 0; } /*=== 5. Login Button ===*/ .login-form-1 .login-button { position: absolute; right: -25px; top: 50%; background: #ffffff; color: #999999; padding: 11px 0; width: 50px; height: 50px; margin-top: -25px; border: 5px solid #efefef; border-radius: 50%; transition: all ease-in-out 500ms; float: right; } .login-form-1 .login-button:hover { color: #555555; transform: rotate(450deg); } .login-form-1 .login-button.clicked { color: #555555; } .login-form-1 .login-button.clicked:hover { transform: none; } .login-form-1 .login-button.clicked.success { color: #2ecc71; } .login-form-1 .login-button.clicked.error { color: #e74c3c; } /*=== 6. Form Invalid ===*/ label.form-invalid { position: absolute; top: 0; right: 0; z-index: 5; display: block; margin-top: -25px; padding: 7px 9px; background: #777777; color: #ffffff; border-radius: 5px; font-weight: bold; font-size: 11px; } label.form-invalid:after { top: 100%; right: 10px; border: solid transparent; content: " "; height: 0; width: 0; position: absolute; pointer-events: none; border-color: transparent; border-top-color: #777777; border-width: 6px; } /*=== 7. Form - Main Message ===*/ .login-form-main-message { background: #ffffff; color: #999999; border-left: 3px solid transparent; border-radius: 3px; margin-bottom: 8px; font-weight: bold; height: 0; padding: 0 20px 0 17px; opacity: 0; transition: all ease-in-out 200ms; } .login-form-main-message.show { height: auto; opacity: 1; padding: 10px 20px 10px 17px; } .login-form-main-message.success { border-left-color: #2ecc71; } .login-form-main-message.error { border-left-color: #e74c3c; } /*=== 9. Misc ===*/ .logo { padding: 15px 0; font-size: 25px; color: #aaaaaa; font-weight: bold; } #btnLogin { float: right; }

### register.css

register.css

1. Create reigster.css in the static/css folder and copy the following codes: a { color: #aaaaaa; transition: all ease-in-out 200ms; } a:hover { color: #333333; text-decoration: none; } /*=== 3. Text Outside the Box ===*/ .etc-login-form { color: #919191; padding: 10px 20px; clear: right; } .etc-login-form p { margin-bottom: 5px; } /*=== 4. Main Form ===*/ .login-form-1 { max-width: 300px; border-radius: 5px; /*display: inline-block;*/ margin: auto; } .main-login-form { position: relative; } .login-form-1 .form-control { border: 0; box-shadow: 0 0 0; border-radius: 0; background: transparent; color: #555555; padding: 7px 0; font-weight: bold; height: auto; } .login-form-1 .form-control::-webkit-input-placeholder { color: #999999; } .login-form-1 .form-control:-moz-placeholder, .login-form-1 .form-control::-moz-placeholder, .login-form-1 .form-control:-ms-input-placeholder { color: #999999; } .login-form-1 .form-group { margin-bottom: 0; border-bottom: 2px solid #efefef; padding-right: 20px; position: relative; } .login-form-1 .form-group:last-child { border-bottom: 0; } .login-group { background: #ffffff; color: #999999; border-radius: 8px; padding: 10px 20px; margin-bottom: 5%; } .login-group-checkbox { padding: 5px 0; } /*=== 5. Login Button ===*/ .login-form-1 .login-button { position: absolute; right: -25px; top: 50%; background: #ffffff; color: #999999; padding: 11px 0; width: 50px; height: 50px; margin-top: -25px; border: 5px solid #efefef; border-radius: 50%; transition: all ease-in-out 500ms; float: right; } .login-form-1 .login-button:hover { color: #555555; transform: rotate(450deg); } .login-form-1 .login-button.clicked { color: #555555; } .login-form-1 .login-button.clicked:hover { transform: none; } .login-form-1 .login-button.clicked.success { color: #2ecc71; } .login-form-1 .login-button.clicked.error { color: #e74c3c; } /*=== 6. Form Invalid ===*/ label.form-invalid { position: absolute; top: 0; right: 0; z-index: 5; display: block; margin-top: -25px; padding: 7px 9px; background: #777777; color: #ffffff; border-radius: 5px; font-weight: bold; font-size: 11px; } label.form-invalid:after { top: 100%; right: 10px; border: solid transparent; content: " "; height: 0; width: 0; position: absolute; pointer-events: none; border-color: transparent; border-top-color: #777777; border-width: 6px; } /*=== 7. Form - Main Message ===*/ .login-form-main-message { background: #ffffff; color: #999999; border-left: 3px solid transparent; border-radius: 3px; margin-bottom: 8px; font-weight: bold; height: 0; padding: 0 20px 0 17px; opacity: 0; transition: all ease-in-out 200ms; } .login-form-main-message.show { height: auto; opacity: 1; padding: 10px 20px 10px 17px; } .login-form-main-message.success { border-left-color: #2ecc71; } .login-form-main-message.error { border-left-color: #e74c3c; } /*=== 8. Custom Checkbox & Radio ===*/ /* Base for label styling */ [type="checkbox"]:not(:checked), [type="checkbox"]:checked, [type="radio"]:not(:checked), [type="radio"]:checked { position: absolute; left: -9999px; } [type="checkbox"]:not(:checked)+label, [type="checkbox"]:checked+label, [type="radio"]:not(:checked)+label, [type="radio"]:checked+label { position: relative; padding-left: 25px; padding-top: 1px; cursor: pointer; } /* checkbox aspect */ [type="checkbox"]:not(:checked)+label:before, [type="checkbox"]:checked+label:before, [type="radio"]:not(:checked)+label:before, [type="radio"]:checked+label:before { content: ''; position: absolute; left: 0; top: 2px; width: 17px; height: 17px; border: 0px solid #aaa; background: #f0f0f0; border-radius: 3px; box-shadow: inset 0 1px 3px rgba(0, 0, 0, 0.3); } /* checked mark aspect */ [type="checkbox"]:not(:checked)+label:after, [type="checkbox"]:checked+label:after, [type="radio"]:not(:checked)+label:after, [type="radio"]:checked+label:after { position: absolute; color: #555555; transition: all .2s; } /* checked mark aspect changes */ [type="checkbox"]:not(:checked)+label:after, [type="radio"]:not(:checked)+label:after { opacity: 0; transform: scale(0); } [type="checkbox"]:checked+label:after, [type="radio"]:checked+label:after { opacity: 1; transform: scale(1); } /* disabled checkbox */ [type="checkbox"]:disabled:not(:checked)+label:before, [type="checkbox"]:disabled:checked+label:before, [type="radio"]:disabled:not(:checked)+label:before, [type="radio"]:disabled:checked+label:before { box-shadow: none; border-color: #8c8c8c; background-color: #878787; } [type="checkbox"]:disabled:checked+label:after, [type="radio"]:disabled:checked+label:after { color: #555555; } [type="checkbox"]:disabled+label, [type="radio"]:disabled+label { color: #8c8c8c; } /* accessibility */ [type="checkbox"]:checked:focus+label:before, [type="checkbox"]:not(:checked):focus+label:before, [type="checkbox"]:checked:focus+label:before, [type="checkbox"]:not(:checked):focus+label:before { border: 1px dotted #f6f6f6; } /* hover style just for information */ label:hover:before { border: 1px solid #f6f6f6 !important; } /*=== Customization ===*/ /* radio aspect */ [type="checkbox"]:not(:checked)+label:before, [type="checkbox"]:checked+label:before { border-radius: 3px; } [type="radio"]:not(:checked)+label:before, [type="radio"]:checked+label:before { border-radius: 35px; } /* selected mark aspect */ [type="checkbox"]:not(:checked)+label:after, [type="checkbox"]:checked+label:after { content: '✔'; top: 0; left: 2px; font-size: 14px; } [type="radio"]:not(:checked)+label:after, [type="radio"]:checked+label:after { content: '\2022'; top: 0; left: 3px; font-size: 30px; line-height: 25px; } /*=== 9. Misc ===*/ .logo { padding: 15px 0; font-size: 25px; color: #aaaaaa; font-weight: bold; } #btnRegister { float: right; }

### style.css

styles.css

1. Create styles.css in the static/css folder and copy the following codes: .row.content { height: 550px } .sidenav { background-color: #212020; color: #c4c2c2; height: 800px; } a { color: white; } #navigations { font-size: 1.5rem; color: white; } .panel>.panel-heading { background-image: none; background-color: #3b3b3b; color: white; } .active { background-color: #3b3b3b; border-radius: 10px; }

smartdoorbellWeb

. ¦ ├── certs ¦ ├── certificate.pem.crt ¦ ├── private.pem.key ¦ ├── public.pem.key ¦ └── rootca.pem ¦ ├── static ¦ └── css ¦ ├── login.css ¦ ├── register.css ¦ ├── styles.css ¦ └── welcome.css ¦ ├── templates ¦ ├── dashboard.html ¦ ├── lcd.html ¦ ├── light.html ¦ ├── login.html ¦ ├── picture.html ¦ ├── register.html ¦ └── welcome.html ¦ └── server.py

# Section 9

# Running

- Web Application

Task

- In the project directory for the web application, run sudo python server.py.

- Smart Doorbell

Task

- In the project directory for the raspberrypi, run sudo python ane_doorbell.py.

-- End of CA2 Step-by-step tutorial --