NA1550 SW UPGRADE PROCEDURE

REMINDER:

MCU SW cannot be upgraded together with DVD SW, namely, these 2SWs can not be copied into one USB at same time; need to detach, and upgrade one by one, DVD SW first, then, MCU SW.

One. DVD SW Upgrade Step:

1) Decompress “TF21K.BIN”to USB (normal using)

2) Install USB to Machine, machine will shown upgrade UI, no need to press any button, 5~6 seconds later, and machine will upgrade automatically.

3) Complete Upgrade, will remind”UPG OK”, switch to TUNER mode, that’s to say Upgrade successfully.

TWO. MCU SW Upgrade Step:

1) Decompress “TF21MCU.BIN”to USB (normal using)

2) Machine better to connect rear video

3) Install USB to machine, machine and rear video will remind”Confirm or not to Upgrade”, press “OK” Key on machine, or “OK” key of remote, Upgrade proceed.

4) Upgrade for MCU SW will take 100 seconds,(will remind in rear video),but, machine will be in black screen, the button light will glitter till upgrade complete; when boot-strap logo shown, switch to audio status, that’s to say Upgrade successfully

Remark: In Upgrade period, pls keep MACHINE POWER to be charged, Power off will make the machine die.