#picaxe 08m

'setfreq m4					'Not necessary at 4MHz - Saves 4 bytes

'	Assign names to pins, variables to registers, and constants.

symbol rnd		=	w6		'Word variable : Current random number

symbol kdel		=	w5		'Word variable : Delay between last and current knock input

symbol cnt1		=	b0

symbol kcnt		=	b1		'Number of knocks

symbol pptr		=	b2		'Pointer to knock delay storage array

symbol tmp1		=	b6		'Reusable variable

symbol tmp2		=	b7		'Reusable variable

symbol mic		=	pin2		'Microphone input (can also be output)

symbol knock		=	4		'Knock motor output

symbol led		=	0		'Indicator LED for testing

symbol pstart	=	$50			'Start of storage area

symbol tmax 	=	800			'Time-out value for delay - About 1.5 seconds

symbol settle	=	80			'Settling time for microphone

'	This section is where the person knocks.

'	The time between successive knocks is stored

' 	until there is a pause of 1.5 seconds or so.

do								'Start of main loop

kcnt = 0						'Initialise knock counter

input 2							'Define mic as input to detect knocks

do

for kdel = 1 to tmax				'KDel is counting loops to determine time between knocks

if mic = 1 then			'Detect HI on microphone input

inc kcnt			'Add 1 to number of knocks

high led			'Flash LED to help debugging

pause settle			'Wait for oscillations to stop

low led

kdel = kdel + settle		'Add settle time to loop counter for accuracy

pptr = kcnt * 2 + pstart	'KCnnt is a word variable so need to add 2 to pointer

poke pptr,word kdel		'Write the delay from last knock to storage area

kdel = 0			'Reset kdel ready for next knock

end if

next kdel

loop until kdel >= tmax				'If no knocks for over TMax loops then leave loop

'This section will operate depending on the number of knocks detected.

'If there are no knocks it will exit.

'Other things happen on 13 or 20 knocks, or it will echo the knocks for any other number.

'The maximum number of knocks is 23 to fill buffer. Incorrect count after that.

select case kcnt

case 0					'Do nothing if no knocks have occurred

case 20					'If 20 knocks play go into prank mode

gosub twoknock

wait 30				'Do nothing for 30 seconds

for tmp1 = 1 to 50		'Make 50 knocks with random spacing

random W6		'Generates next random word

tmp2 = rnd + 100	'Make delay value from random value

gosub doknock		'Make a knock

pause tmp2		'Pause for the random delay number of milliseconds

next tmp1			'Loop until 50 knocks done

case 13					'Piecax's favourite number

tune 0, 7,($50,$35,$79,$35,$70,$0B,$F7,$75,$34,$77,$34,$70,$09,$F5,$70,$35,$79,$35,$70,$0B,

$F7,$75,$34,$70,$32,$74,$B5,$70,$72,$74,$75)

gosub twoknock				'TUNE command plays the series of bytes as notes

tune 0, 7,($72,$74,$76,$77)

gosub twoknock				'TWOKNOCK knocks twice with suitable delays between

tune 0, 7,($72,$74,$76,$77,$3C,$72,$74,$76,$77,$3C,$70,$72,$74,$75)

gosub twoknock

else 					'Any other number of knocks will be echoed

for tmp1 = 1 to kcnt			'Loop for the number of knocks

pptr = tmp1 * 2 + pstart	'Step in twos through buffer

peek pptr,word kdel		'Read back delay value words from buffer

pause kdel			'Wait the delay time (in milli-seconds)

gosub DoKnock			'Perform a knock

next tmp1				'Loop for next knock

pause 200

endselect

loop

'	This section contains the knocking routines

DoKnock:

high knock					'Turn on motor

pause 70					'Wait 70 milli-seconds

low knock					'Turn off motor

return

TwoKnock:						'More compact to do this as subroutine

pause 550					'Two knocks with timed delays between for tune

gosub doknock

pause 700

gosub doknock

pause 700

return