/* The sample code was created by Ryan Winters for an Exterior Light project

for Jameco Electronics and Club Jameco. Use this code to test the function of the

sensors and also to record values reported by the sensors using the Serial Monitor.

************************************************************************

Relay signal should be connected to digital pin 3 (ATmega328 physical pin 5)

PIR signal wire should be connected to digital pin 2 (ATmega328 physical pin 4)

Photocell signal should be connected to analog input 0 (ATmega328 physical pin 23)

Status LED should be connected to digital pin 13 (ATmega328 physical pin 19)

Status LED is already built into the Ardweeny (still digital pin 13, physical pin 19)

***********************************************************************/

int Relay = 3;                                		//relay is on digital pin 3

void setup()

{

Serial.begin(9600);                         		//setup the Serial monitor

Serial.println("Warming up...");            	//display text while waiting for PIR to calibrate

delay(20000);                               		//wait 20 sec required by PIR for calibration

pinMode(13,OUTPUT);                         	//additional LED for relay status indicator set as output

pinMode(Relay,OUTPUT);                      	//set relay as an output

}

void loop()

{

int Light = analogRead(0);                  	//stores value read from photocell on analog 0

Serial.println(Light);                      		//print photocell value

Serial.println(digitalRead(2),DEC);         	//print PIR value

delay(500);                                 		//wait 1/2 second and loop

}