Abbreviated Pre-Trip Truck Inspection Check List

20-Foot Flatbed Truck, with Lift-Gate and Optional Stake Walls:

Driver Side, Notes:																				     Passenger Side, Notes:

Supervisor:		###-###-####

Warehouse:		###-###-####					Vertical Clearance Required For Vehicle (measure and note here):

Destination notes in margin 

Event / Company, Project Manager  Address, City, State, Zip Code

Date: ___________		Driver: ____________________	Odometer: ______________		Destination(s):

⁪ Battery Last Checked: _____

⁪ License Plate Date: _______

⁪ Engine Oil / Wiper Wash

⁪ Brake / Transmission Fluids

⁪ Fan Belts

⁪ Coolant / Hoses

⁪ Leaf Springs: No Cracks

⁪ Tire Pressures / Condition

⁪ Drive Shaft / U-Joint

⁪ Undercarriage / Mud Flaps

⁪ Lift-Gate Functions

⁪ Load & Rigging Secure

⁪ Fire Extinguisher, Etc

⁪ Horn

⁪ Windshield Wipers

⁪ Headlights: Dim-Bright

⁪ Turn Signals / Brake Lights

⁪ Emergency Flasher Test

⁪ Steering: No Loose Play

⁪ Reverse Lights / Beeper

⁪ Ratchet Straps (hook only)

⁪ Insurance Expires: ________

⁪ Registration Exp: _________

⁪ Windows & Mirrors Clean

⁪ FasTrak / CFN Fuel Card

⁪ Cell Phone is Hands-Free

⁪ GPS is Hands-Free

⁪ Flashlight, Area Floodlight

⁪ Dashboard Gauges Function

⁪ * Axle Weights (see below)

⁪ ** Fuel Status (see below)

⁪ Brakes / Parking Brakes

⁪ Condition of vehicle is satisfactory, and within weight limits. Cargo is adequately secured.

GVWR:          26,000# (= 13,940# max payload)

Front GAWR:  7,000# (=   1,260# max payload = 1/10th rear axle payload)

Rear GAWR: 19,000# (= 12,680# max payload = 10x front axle payload)

* Formula for determining approximate axle weights:

Front Axle: 14,103 - ( 77.9 × avg leaf spring-to-frame mm)

Rear Axle:  82,362 - (209.5 × avg leaf spring-to-frame mm)

Front Axle (7,000# max)						Rear Axle (19,000# max)

Left-Front:											Left-Rear:

_______mm										_______mm

Right-Front:											Right-Rear:

_______mm										_______mm

= Average:											= Average:

_______mm										_______mm

14,103														82,362

- (77.9 × avg mm) 				  	 	- (209.5 × avg mm)

Front Axle = ___________#		 		Rear Axle = __________#

A convenient reference point:

Public Scale Weight, Empty, with 7/8 tank of fuel:  12,060#

Steer (Front):    5,740# (107.50mm avg leaf spring-to-frame)

Drive (Rear):    6,320# (363.00mm avg leaf spring-to-frame)

Center of gravity of payload (assumed forward of rear axle):

(Front Axle# - 5,740#) + (Rear Axle# - 6,320#) = Total Payload#

(Front Axle Payload# ÷ Total Payload#) × 237.25" Wheelbase

= Center of Gravity, measured in inches, forward of rear axle

To shift load: ÷ desired Front Axle Payload# by Total Payload#

Common load densities:

Diesel fuel weighs 7.15# per U.S. gallon x 100 gallons = 715#

Aluminum stagelight trusses weigh 5# to 7# per cubic foot

Solid lumber and plywood weighs 35# per cubic foot

Concrete weighs 147# per cubic foot; Stones 104# per cubic foot

1998 Ford F800 F Series

VIN: #XXXX##X#XXX#####

California License Plate Number: #X#####

Category:	Flatbed Truck

Style / Body: Conventional Cab

Front Disk Brakes, Drum Rear Brakes

Tires: 10R 22.5F, 100psi cold

Engine: ISB Series, 5.9L (= 359 CID), L6, Diesel

Flatbed Max. Dimensions: 21' 2.5" long × 8' 0" wide

Flatbed Height (empty): 4' 3" = loading dock height

Flatbed Capacity: 13,900# maximum payload

Lift-Gate Capacity: 2,500 lbs

** Dual 50-Gallon Fuel Tanks

Total Truck Length: 30' 2.5"

Wheel Axles Wheelbase:  19' 91/4" (= 237.5")

GVWR Type: Class 6 (GVW 26,000# max)

**	Mileage: 10.9mpg highway driving @ 40% payload capacity; 7.2mpg city driving @ 40% payload capacity

**	Each fuel tank holds 50 gallons of diesel fuel, but only 46.5 gallons is usable. Step-tanks located below driver and passenger doors

**	Onboard fuel gauge understates remaining fuel:  Use calibrated wooden fuel dipstick!

Fuel dipstick (11/16" wooden dowel, 213/8" long) calibration, in U.S. gallons, for 50-gallon step tanks:

0 gal.= 31/32"; 5= 21/8";  10= 37/32"; 15= 45/8"; 20= 513/16"; 25= 75/16"; 30= 823/32"; 35= 931/32"; 40= 113/8"; 45= 123/4"; 46.5= 137/32"

For More Information:		http://www.instructables.com/id/How-to-Estimate-the-Axle-Weights-of-a-Standard-2-A/