Flip sign for flat-panel monitor

- Pick out green laminated sheets. Printed sheets will be bottom pieces--it's okay to have dirty specks. Sheets that are blank on both sides will be top pieces, and they need to be mostly clean on both sides.

- Trim off side with punched holes.

- Cut in half (make two 5.5 x 8.5 pieces).

- Separate into pieces with print (these will be bottoms) and pieces blank on both sides (these will be tops).

- Take one top and one bottom and clip them about 1" apart on both sides (outside the green on the thin stencils).

- Connect the top and bottom with 2 strips of CLEAR 1.5"- 2" wide strapping tape—about 4 to 5" long. Turn the pieces over and do the same on the back, so that no sticky part is exposed.

- Use the thick stencil to mark a slit on both sides of the bottom piece. Cut with Xacto or scissors. It's okay if the slit is over the tape—that will provide reinforcement.

- Print out the Login Required.doc file.

- Trim signs close to borders.

- Tape signs centered on the front and back of the top piece, oriented carefully.

- Cut the stiff plastic in half at the bottom of the "U".

- Curve the long end over and staple at the bottom.

- Use the thick stencil to cut the base at an angle. The top of the angle should be on the side opposite the "L" tab.

- Take the piece you just cut off, turn it over to match the angle of the base, and staple the sides separately to the outside of the base.

- Cut down the middle of the "L" tab to where it meets the base.

- Bend these new tabs in opposite directions at 90-degrees to the base.

- Bend the tabs back together, stick them through the slits in the bottom piece, so that the tabs come out on the printed side.

- Bend the tabs flat on the bottom piece, and staple securely.

- Tape the bottom piece securely to the back of the flat-panel monitor using blue painter's tape (so it doesn't leave sticky residue).

- Check that the signs are correct in the up and down positions.

http://www.instructables.com/id/Flip-sign-for-flat-panel-monitor/	12/17/07 SN