|  | Simulation of upper arm Date: zondag 9 oktober 2016 Designer: Teun Study name: SimulationXpress Study Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Material Properties 3 Loads and Fixtures 3 Mesh Information 4 Study Results 6 Conclusion 8. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: upper arm Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Cut-Extrude3; Solid Body; Mass:0.10574 kg Volume:1.37324e-005 m^3 Density:7700 kg/m^3 Weight:1.03625 N; D:\Documents\Industrieel Ontwerper\Jaar 3\Bachelor opdracht\cad\pack and go 2\upper arm.SLDPRT Oct 07 11:55:50 2016.

# Material Properties Model Reference; Properties; Components. Name:; Alloy Steel. Model type:; Linear Elastic Isotropic. Default failure criterion:; Max von Mises Stress. Yield strength:; 620.422 N/mm^2. Tensile strength:; 723.826 N/mm^2.; SolidBody 1(Cut-Extrude3)(upper arm).

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 1 face(s). Type:; Fixed Geometry.. Load name; Load Image; Load Details. Force-1; Entities:; 1 face(s). Type:; Apply normal force. Value:; 5 N..

# Mesh Information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points; 4 Points. Element Size; 2.57151 mm. Tolerance; 0.128575 mm. Mesh Quality; High. ## Mesh Information - Details Total Nodes; 23219. Total Elements; 11608. Maximum Aspect Ratio; 15.376. % of elements with Aspect Ratio < 3; 86.4. % of elements with Aspect Ratio > 10; 0.0517. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:05. Computer name:; TEUN-PC.

# Study Results Name; Type; Min; Max. Stress; VON: von Mises Stress; 0.00302417 N/mm^2 (MPa) Node: 217; 38.9325 N/mm^2 (MPa) Node: 910. upper arm-SimulationXpress Study-Stress-Stress. Name; Type; Min; Max. Displacement; URES: Resultant Displacement; 0 mm Node: 15; 0.0742052 mm Node: 240. upper arm-SimulationXpress Study-Displacement-Displacement. Name; Type. Deformation; Deformed Shape. upper arm-SimulationXpress Study-Displacement-Deformation. Name; Type; Min; Max. Factor of Safety; Max von Mises Stress; 15.9359 Node: 910; 205154 Node: 217. upper arm-SimulationXpress Study-Factor of Safety-Factor of Safety.

# Conclusion